EE 552

Advance Entry System

Project Specifications

Group:

Patrick Li 351018

Edgar Wong 356898

Edward Wong 263923

Howard Shum 338714

Professor: Dr. D. Elliott

Date: October 6,1998

TABLE OF CONTENTS

INTRODUCTION	2

MODULE	2

LCD Display Module	2

Keypad Module	2

Motor Module	2

RAM	2

Internal Menu Module	2

OTHER DESIGN CONSIDERATIONS	3

Entity controller	3

Schematic:	4

APPENDICES	5

Time Management	5

High Level VHDL (entity)	6

# INTRODUCTION

This project includes the design and implementation of the time schedule Controller.  The controller is designed to control a electric motor which attached to a advanced entry system.  This controller provides user a option of scheduling the operation of the controller itself.  The design of the controller is mainly consisted of a keypad (input), LCD Display (Output) and the processing unit is design using the Altera chip.

# MODULE

## LCD Display Module

Display the clock, on screen menu and the schedule on a LCD display requires 14 I/O pins.

## Keypad Module

An interface for user to entry various functions of the controller. Since the keypad has 14-pin outputs, it needs 14 pins on the FPGA chip.

## Motor Module

A Stepper Motor is used to control the angle of controlling device. This will need 4 I/O pins.

## RAM

The ram uses to storage of the schedule for the operating period on the controller and password to use the system. This system will use a 256K to store all data. It will take 16 I/O pins.

Clock Module

Display the time, day and provide the time of scheduling.  Default is set to the actual time

## Internal Menu Module

Provide a friendly interface for user to set option on the controller, and all functions to administrator.

Optional Modules Add On

- Sensor			: Use to provide a feedback to the controller

- Ethernet Hookup		: Use to transmit schedule and logs to a computerized database and provide remote administration through the internet

- Custom Password		: Use to allow user to change to their own password

- Backup power		: On the main power failure, a backup power is provided

# OTHER DESIGN CONSIDERATIONS

| TESTING | To test our system, we will construct the system on a Altera board and use different switches or input signals to simulate all conditions of the system separately on each module. Therefore we can match the output to our goal easily and debug any errors |
| --- | --- |
| Additional Parts | For this controller system, we requires a 2 lines by 16 characters wide LCD display, 12 buttons 0-9 keypad, Stepper motor, and 256K RAM |
| Optional Parts | These parts will be used if time allowed. An infrared sensor for detecting motion, for example, the system is used to control an automatic entrance, or mass transit system automatic tollgates. Ethernet can be connected to mainframe computer to control the whole system remotely, and store all log files for research and improvement. A DP83902A, Twisted pair Serial Network Internet Controller ( ST-NIC) can be use for this option A switching circuit and batteries are for state of emergency of electricity blackout or power supply failure. |
| Chip | We will use Altera chip because it is re-programmable and it has high density and provides large number of gates for more option to add on. |

# Entity controller

Entity FPGA

Entity RAM

Entity display

Entity keypad

# Altera FPGA

256K Ram

# Schematic:

# Display LCD

Keypad

# Stepper Motor

# Altera FPGA

256K Ram

# APPENDICES

## Time Management

| Task | Deadline | Person | Duration | Test/Debug |
| --- | --- | --- | --- | --- |
| Project Specification | Oct. 6 | Group | 6 hours | None |
| Design | Oct. 10 | Group | 18 hours | None |
| Resource Requirement | Oct. 27 | Edward | 5 hours | None |
| Application Notes | Oct. 29 | Edgar | 4 hours | None |
| Code Developing | Nov. 2 | Group |  |  |
| Clock VHDL |  |  | 8 hours | 4 hours |
| I/O VHDL |  |  | 16 hours | 8 hours |
| Controller VHDL |  |  | 20 hours | 10 hours |
| Optional VHDL |  |  | 30 hours | 15 hours |
| Code Testing | Nov. 3 | Howard/Edgar | 15 hours | 15 hours |
| Simulation Documentation | Nov. 10 | Patrick | 10 hours | None |
| Hardware Connection | Nov. 13 | Edward/Patrick | 15 hours | 5 hours |
| Device Testing | Nov. 20 | Group | 10 hours | 5 hours |
| Final Report | Nov. 23 | Group | 10 hours | None |
| Oral Presentation | Nov. 24 Dec. 1, 3 | Group | 10 minutes | 2 hours |

## High Level VHDL (entity)

VHDL:

Entity LCD_display is

Port(LCD_output : buffer std_logic_vector(7 downto 0);

LCD_sigin : buffer std_logic_vector(7 downto 0);

RS : out std_logic;

R/W : out std_logic;

Clock : out std_logic);

End LCD_display;

Description:

This function was used to initialize and display the menu of the LCD.

LCD_output : 8bits signal output to the LCD display. The signal contains the necessary code to display in the LCD screen.

LCD_sigin : 8 bits signal input to the LCD display. The function receives the data from other function to display on the LCD screen.

R/W : The signal which will select the LCD in read or write mode. When R/W is 0, it enable Altera controller write data to the LCD. When R/W is 1, it enables Altera controller send data to the LCD.

RS : The signal to select registers. 0 for instruction register and 1 for data register.

Clock : clock signal for the LCD.

Entity menu is

Port(menu_output : buffer std_logic_buffer(7 downto 0);

Menu_input : buffer std_logic_buffer(7 downto 0);

Menu select : in std_logic_buffer(3 downto 0);

Clock : in std_logic);

End menu;

Description:

This function contain the necessary message for the LCD display. It can also received the external data to display on the LCD screen.

Menu_output : an 8bit signal output to the LCD_display which contain the message of the LCD screen.

Menu_input : an 8bit signal input to the menu for updating the message (time and data).

Menu_select : a signal which enable the LCD display up to 15 messages.

Clock : an input clock signal for the menu function.

Entity timer is

Port(  pass1 : in std_logic(1 downto 0);

sethour, setminute, setmonth, setday : buffer std_logic_vector(7 downto 0);

Month_display : buffer std_logic(7 downto 0);

Day_display : buffer std_logic(7 downto 0);

Hour_display : buffer std_logic(7 downto 0);

Minute_display : buffer std_logic(7 downto 0);

Second_display : buffer std_logic(7 downto 0);

Clock out std_logic);

End timer;

Description:

This function was used to display the time and day on the LCD screen. It also enables the user to set the time and day with permission.

Pass1 : a 2bit signal which allow the user to set the time and day. '01' is enable and others is disable. The reason for using two bits instead of one bit is for safety purpose.

Sethour : an 8bit input from the keypad decoder to set the hour.

Setminute : an 8bit input from the keypad decoder to set the minute.

Setmonth : an 8bit input from the keypad decoder to set the month.

Setday : an 8bit input from the keypad decoder to set the day.

Month_display : an 8bit data of month to display in LCD.

Day_display : an 8bit data of day to display in LCD.

Hour_display : an 8bit data of hour to display in LCD.

Minute_display : an 8bit data of minute to display in LCD.

Second_display : an 8bit data of second to display in LCD.

Clock : a clock signal provide to the whole project.

Entity keypad is

Port( keyin : in std_logic_vector(11 downto 0);

Keyout : out std_logic_vector(3 downto 0);

Clock : in std_logic);

End keypad

Description:

The function of keypad used to input the signal from the keypad and decode it into number digit.

Keyin : There is 12 bottom on the keypad which require 12 bit inputs.

Keyout : a 4bit output decoded number which corresponding to the each key.

Clock : a clock signal used to perform de-bouncing.

Entity store is

Port(selectchip : buffer std_logic_vector(7 downto 0);

dataout : buffer std_logic_vector(15 downto 0);

A/D : buffer std_logic;

R/Wchip : out std_logic;

Data : buffer std_logic_vector(7 downto 0);

R/Wstore : in std_logic);

End store;

Description:

This function will receive the data such as encrypted password and time, day of entry from the caller function and store into the memory such as ram. Also, the R/Wstore pin enables the caller function to get data from the memory.

Selectchip : an 8bits signal enable the function to select up to 256 of ram chip for memory storage.

Dataout : an 16bits signal contain either data or address to memory storage.

A/D : this pin select the Dataout to be address or data.

R/Wchip : this pin select the data input or data output.

Data : an 8bits signal used to send and receive for the store function.

R/Wstore : a pin to select the data in and out of the store function.

Entity encrypted is

Port(data_in : in std_logic_vector(7 downto 0);

Data_out : out std_logic_vector(7 downto 0);

Enable : in std_logic;

En/Disen : in std_logic);

End encrypted;

Description:

This function encrypts the user password and store into the memory or restores the encrypted password from the memory.

Data_in : an 8bits data input to the encrypt function.

Data_out : an 8 bits data output from the encrypt function.

Enable : this pin enable the function to perform encryption and restoration.

En/Disen : this pin select the function to perform encryption or restoration.

Entity pass_confirm is

Port(data_in : in std_logic_vector(7 downto 0);

Data1_pass : in std_logic_vector(7 downto 0);

Signal : out std_logic_vector(3 downto 0);

End pass_confirm;

Description:

This function will input the password from the keypad and the restore password from the memory. Then compare both password and output the confirm signal.

Data_in : an 8 bits data of the password from the keypad.

Data1_pass : an 8 bits data of the password from the memory.

Signal : an 4 bit confirmed signal use for control.

Entity motor is

Port(enable : in std_logic;

Output4 : out std_logic;

Clock : in std_logic);

End motor;

Description:

This function control the stepper-motor which simulate a automatic door.

Enable : a pin enable the motor to turn.

Output4 : a 4bit output signal to the motor.

Clock : a input clock signal use for counting.