EE 552

Advance Entry System

Resource Requirement

Group:

Patrick Li 351018

Edgar Wong 356898

Edward Wong 263923

Howard Shum 338714

Professor: Dr. D. Elliott

Date: October 27,1998

Table of contents

INTRODUCTION	2

MODULE and VHDL code	2

LCD Display Module	2

VHDL code: compiles without errors	2

Keypad Module	4

VHDL code: compiles without errors	4

Clock Module	6

VHDL code: compiles without errors	6

Time Management	9

Last handed-in schedule	9

Updated detail schedule	9

Estimate the total number of logic blocks	10

List of test cases	10

# INTRODUCTION

This project includes the design and implementation of the time schedule Controller.  The controller is designed to control an electric motor, which attached to a advanced entry system.  This controller provides user a option of scheduling the operation of the controller itself.  The design of the controller is mainly consisted of a keypad (input), LCD Display (Output) and the processing unit is design using the Altera chip.

# MODULE and VHDL code

## LCD Display Module

Display the clock, on screen menu and the schedule on a LCD display requires 14 I/O pins.

The code for this part has been generated and tested. Due to the leak of other code connection, it is hard to say it works properly with the whole system at this moment.

### VHDL code: compiles without errors

--lcd_init.vhd This program initialize the LCD display

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity lcd_init is

generic(width : positive := 8);

port(reset, clock, enable, done_ack : in std_logic;

done, RW, rs, lcd_e : out std_logic;

Qout : buffer std_logic_vector(width-1 downto 0)

);

end lcd_init;

architecture initial of lcd_init is

type state_type is (waiting, i0, i1, i2, i3, donestate);

signal state, next_state : state_type;

signal done_temp, lcd_e_temp : std_logic;

signal count,count_temp : std_logic_vector(1 downto 0);

signal Qout_temp : std_logic_vector(width-1 downto 0);

begin

running : process(state,enable,done_ack,count) is

begin

case state is

when waiting =>

done_temp <= '0';

lcd_e_temp <= '0';

RW <= '0';

rs <= '0';

Qout_temp <= "00000000";

if enable = '1' then

next_state <= i0;

else

next_state <= waiting;

end if;

when i0 =>

Qout_temp <= "00000001";

lcd_e_temp <= '1';

done_temp <= '0';

if count = "11" then

next_state <= i1;

else

next_state <= i0;

end if;

when i1 =>

Qout_temp <= "00000010";

lcd_e_temp <= '1';

done_temp <= '0';

if count = "11" then

next_state <= i2;

else

next_state <= i1;

end if;

when i2 =>

Qout_temp <= "00111000";

lcd_e_temp <= '1';

done_temp <= '0';

if count = "11" then

next_state <= i3;

else

next_state <= i2;

end if;

when i3 =>

Qout_temp <= "00001110";

lcd_e_temp <= '1';

done_temp <= '0';

if count = "11" then

next_state <= donestate;

else

next_state <= i3;

end if;

when donestate =>

done_temp <= '1';

lcd_e_temp <= '1';

Qout_temp <= ( others => '0' );

if done_ack = '1' then

next_state <= waiting;

else

next_state <= donestate;

end if;

end case;

end process running;

timing : process(clock,reset) is

begin

if rising_edge(clock) then

Qout <= Qout_temp;

count <= count_temp;

lcd_e <= lcd_e_temp;

done <= done_temp;

if reset = '1' then

state <= waiting;

count_temp <= "00";

else

state <= next_state;

count_temp <= count_temp + "01";

end if;

end if;

end process timing;

end initial;

## Keypad Module

An interface for user to entry various functions of the controller. Since the keypad has 14-pin outputs, it needs 14 pins on the FPGA chip.

Since the simulation of keypad has been limited, we can't test it with clock input. The code can be compiled but a complete testing will require an oscilloscope. Therefore, we need to access Labs on the 3rd floor to have it done.

### VHDL code: compiles without errors

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity keypad is --princess auto keypad

port(

keyin: in std_logic_vector (6 downto 0); --pin 3 to pin 9 on keypad

a,b: out std_logic; --pin A and pin B on keypad

clock,reset: in std_logic;

keyout: out std_logic_vector (11 downto 0)); -- active low output

end entity;

--key: 0  1  2  3  4  5  6  7  8  9  up  down

--	   a6 b6 a5 b5 a4 b4 a3 b3 a2 b2 a1  a0

architecture structure of keypad is

signal deb_key: std_logic_vector (6 downto 0); --debounced keyin

signal a_detect,b_detect: std_logic; --row A detected a key, row B detected a key

signal deb_reset,keyPress: std_logic; --keyPress: a key is pressed, deb_reset: for resetting the debounce circuit

type statetype is (initialize,scan_a,scan_b);

signal state, next_state: statetype;

begin

--debounce circuit (combinational part)

--set/reset active low flipflop, use "keyin" to set, and "deb_reset" to reset the ff.

--with asychronous reset

deb_key(6) <= ((deb_key(6) nand keyin(6)) nand deb_reset) or reset;

deb_key(5) <= ((deb_key(5) nand keyin(5)) nand deb_reset) or reset;

deb_key(4) <= ((deb_key(4) nand keyin(4)) nand deb_reset) or reset;

deb_key(3) <= ((deb_key(3) nand keyin(3)) nand deb_reset) or reset;

deb_key(2) <= ((deb_key(2) nand keyin(2)) nand deb_reset) or reset;

deb_key(1) <= ((deb_key(1) nand keyin(1)) nand deb_reset) or reset;

deb_key(0) <= ((deb_key(0) nand keyin(0)) nand deb_reset) or reset;

keyPress <= not ((((((deb_key(6) and deb_key(5)) and deb_key(4)) and deb_key(3)) and deb_key(2)) and deb_key(1)) and deb_key(0));

--debounce circuit (delay part)

--once a key is pressed ("KeyPress" = '1') then set "deb_reset" = '1', delay 1ms, then set "deb_reset" = '0'

delay: process(keyPress,clock,reset)

variable count: std_logic_vector(14 downto 0):=(others => '0');

begin

if reset = '1' then

deb_reset <= '1';

elsif rising_edge(clock) then

if keyPress = '1' then

if count = "110001001010111" then

deb_reset <= '0';

count:= "000000000000000";

else

deb_reset <= '1'; -- a key is being pressed

count := count + 1;

end if;

else

count:= "000000000000000";

end if;

end if;

end process;

--detect what key is being pressed---- a key on the "a" row or a key on the "b" row

--if an "a" row key is detected then "a_detect" = '1'

--if a "b" row key is detected then "b_detect" = '1'

detect_key: process (state,keyPress)

begin

case state is

when initialize =>

a <= '1';

b <= '1';

a_detect <= '1';

b_detect <= '1';

next_state <= scan_a;

when scan_a =>

b <= '1';

a <= '0';

if keyPress = '1' then

a_detect <= '0';

next_state <= scan_a;

else

a_detect <= '1';

next_state <= scan_b;

end if;

when scan_b =>

a <= '1';

b <= '0';

if keyPress = '1' then

b_detect <= '0';

next_state <= scan_b;

else

b_detect <= '1';

next_state <= scan_a;

end if;

end case;

end process;

clocking: process (clock,reset)

begin

if reset = '1' then

state <= initialize;

elsif rising_edge(clock) then

state <= next_state;

end if;

end process;

--key: 0  1  2  3  4  5  6  7  8  9  up  down

--	   a6 b6 a5 b5 a4 b4 a3 b3 a2 b2 a1  a0

-- determine which key is being pressed

keyout <= 	"011111111111" when (deb_key(6) nor a_detect) = '1' else --0

"101111111111" when (deb_key(6) nor b_detect) = '1' else --1

"110111111111" when (deb_key(5) nor a_detect) = '1' else --2

"111011111111" when (deb_key(5) nor b_detect) = '1' else --3

"111101111111" when (deb_key(4) nor a_detect) = '1' else --4

"111110111111" when (deb_key(4) nor b_detect) = '1' else --5

"111111011111" when (deb_key(3) nor a_detect) = '1' else --6

"111111101111" when (deb_key(3) nor b_detect) = '1' else --7

"111111110111" when (deb_key(2) nor a_detect) = '1' else --8

"111111111011" when (deb_key(2) nor b_detect) = '1' else --9

"111111111101" when (deb_key(1) nor a_detect) = '1' else --up

"111111111110" when (deb_key(0) nor a_detect) = '1' else --down

"111111111111";

end structure;

## Clock Module

Display the time, day and provide the time of scheduling. Default is set to the actual time

### VHDL code: compiles without errors

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity realtime is

port(

clock: in std_logic; --25.175 MHz clock

reset: in std_logic;

--setclock: in std_logic_vector (15 downto 0); --hour:minute

time: out std_logic_vector (23 downto 0));--hour:minute:second

end realtime;

architecture structure of realtime is

signal oneKHzP: std_logic; -- 1 KHz pulse

signal oneHzP: std_logic; -- 1 Hz pulse

signal secondA,minuteA,hourA: integer range 9 downto 0 := 0;

signal secondB,minuteB: integer range 5 downto 0 := 0;

signal hourB: integer range 2 downto 0 := 0;

signal secA,secB,minA,minB,hoA,hoB: std_logic_vector ( 3 downto 0):=(others => '0');

begin

-- produce 1 KHz pulse

KHzClk: process(clock)

variable count: std_logic_vector (14 downto 0):=(others => '0');

begin

if rising_edge(clock) then

if count = "110001001010111" then

oneKHzP <= '1';

count := "000000000000000";

else

oneKHzP <= '0';

count := count + '1';

end if;

end if;

end process;

-- produce 1 Hz pulse

HzClk: process(oneKHzP)

variable count: std_logic_vector (9 downto 0):=(others => '0');

begin

if rising_edge(oneKHzP) then

if count = "1111101000" then

oneHzP <= '1';

count := "0000000000";

else

oneHzP <= '0';

count := count +'1';

end if;

end if;

end process;

-- produce second

sec_partA: process(oneHzP,reset)

begin

if reset = '1' then

secondA <= 0;

elsif rising_edge(oneHzP) then

secondA <= secondA + 1;

end if;

end process;

sec_partB: process(secondA,reset)

begin

if reset = '1' then

secondB <= 0;

elsif secondA = 0 then

secondB <= secondB + 1;

end if;

end process;

min_partA: process(secondB,reset)

begin

if reset = '1' then

minuteA <= 0;

elsif secondB = 0 then

minuteA <= minuteA + 1;

end if;

end process;

min_partB: process(minuteA,reset)

begin

if reset = '1' then

minuteB <= 0;

elsif minuteA = 0 then

minuteB <= minuteB + 1;

end if;

end process;

hour_partA: process(minuteB,hourB,reset)

begin

if reset = '1' then

hourA <= 0;

elsif minuteB = 0 then

hourA <= hourA + 1;

elsif hourB =0 then

hourA <= 0;

end if;

end process;

hour_partB: process(hourA,reset)

begin

if reset = '1' then

hourB <= 0;

elsif hourA = 0 then

hourB <= hourB + 1;

elsif hourB = 2 then

if hourA = 4 then

hourB <= 0;

end if;

end if;

end process;

secA <= conv_std_logic_vector(secondA,4);

secB <= conv_std_logic_vector(secondB,4);

minA <= conv_std_logic_vector(minuteA,4);

minB <= conv_std_logic_vector(minuteB,4);

hoA <= conv_std_logic_vector(hourA,4);

hoB <= conv_std_logic_vector(hourB,4);

time(3 downto 0) <= secA;

time(7 downto 4) <= secB;

time(11 downto 8) <= minA;

time(15 downto 12) <= minB;

time(19 downto 16) <= hoA;

time(23 downto 20) <= hoB;

end structure;

All other VHDL modules are still working.

# Time Management

## Last handed-in schedule

| Task | Deadline | Person | Duration | Test/Debug | Completion date |
| --- | --- | --- | --- | --- | --- |
| Project Specification | Oct. 6 | Group | 6 hours | None | Oct 6 |
| Design | Oct. 10 | Group | 18 hours | None | Oct 10 |
| Resource Requirement | Oct. 27 | Edward | 5 hours | None | Oct 26 |
| Application Notes | Oct. 29 | Edgar | 4 hours | None | Oct 27 |
| Code Developing | Nov. 2 | Group |  |  |  |
| Clock VHDL |  |  | 8 hours | 4 hours | Processing |
| I/O VHDL |  |  | 16 hours | 8 hours | Processing |
| Controller VHDL |  |  | 20 hours | 10 hours | Processing |
| Optional VHDL |  |  | 30 hours | 15 hours | Processing |
| Code Testing | Nov. 3 | Howard/Edgar | 15 hours | 15 hours | Processing |
| Simulation Documentation | Nov. 10 | Patrick | 10 hours | None | Processing |
| Hardware Connection | Nov. 13 | Edward/Patrick | 15 hours | 5 hours | Processing |
| Device Testing | Nov. 20 | Group | 10 hours | 5 hours | Processing |
| Final Report | Nov. 23 | Group | 10 hours | None | Processing |
| Oral Presentation | Nov. 24 Dec. 1, 3 | Group | 10 minutes | 2 hours | Processing |

## Updated detail schedule

| Task | Start Date | Proposed completion date | Person | Duration | Completion |
| --- | --- | --- | --- | --- | --- |
| Master program | October 31 | November 7 | Edward | 10hrs | Processing |
| Keypad VHDL | October 15 | October 25 | Patrick | 4hrs | Processing |
| Data Encryption VHDL | October 15 | October 25 | Howard | 4hrs | Processing |
| Clock VHDL | October 15 | October 25 | Edgar | 5hrs | Processing |
| LCD display connection VHDL | October 15 | October 25 | Edward | 3hrs | Processing |
| LCD menu VHDL | October 31 | November 6 | Howard | 3hrs | Pending |
| LCD initialization VHDL | October 15 | October 25 | Edward | 2hrs | Processing |
| Stepper motor VHDL | October 31 | November 6 | Edgar | 2hrs | Pending |
| RAM VHDL | October 31 | November 6 | Edgar | 3hrs | Pending |
| Sensor VHDL | October 31 | November 6 | Patrick | 2hrs | Pending |
| Get password VHDL | October 25 | November 1 | Edward | 2hrs | Starting |
| Check password VHDL | October 25 | November 1 | Edgar | 2hrs | Starting |
| RAM-password VHDL | October 25 | November 1 | Patrick | 2hrs | Starting |
| Change password VHDL | October 25 | November 1 | Howard | 2hrs | Starting |
| Cursor VHDL | October 25 | November 1 | Patrick | 2hrs | Starting |
| Store-time (in-out record) VHDL | October 31 | November 7 | Howard | 2hrs | Pending |
| Display log file VHDL | October 31 | November 7 | Edward | 2hrs | Pending |
| Display current time VHDL | October 31 | November 7 | Patrick | 2hrs | Pending |
| LCD connection | October 31 | November 7 | Howard | 1hrs | Pending |
| Stepper motor connection | October 31 | November 7 | Howard | 1hrs | Pending |
| Exit VHDL | October 31 | November 7 | Patrick | 1hrs | Pending |
| Keypad buffer VHDL | October 31 | November 7 | Patrick | 2hrs | Pending |
| Keypad connection | October 31 | November 7 | Howard | 1hrs | Pending |
| Keypad decoder VHDL | October 31 | November 7 | Patrick | 3hrs | Pending |
| Code testing | October 31 | November 7 | Edgar/ Edward | 5hrs | Pending |

# Estimate the total number of logic blocks

We need 40 to 50 logic cells for each individual VHDL. Therefore, we have at least 15 individual VHDL programs.

Total cells are 750.

# List of test cases

LCD

For the LCD, we perform the following test

- we first compile the code and see if anything display on the LCD

- we then test the LCD code by going into different stages of our design, for example, the LCD should Display the present time and the Text "Enter Password" when no key is entered.

- During the check for Admin or user stages, when the Admin input is high, the LCD should display a menu.

KeyPad

For the KeyPad, we perform the following test

- Initially, the KeyPad is at a waiting stage. It waits for any key input, it the key_input is high, it will jump to the key buffer stage, otherwise, it will stay in the waiting stage

- As the key pad is triggered by any key pressed, the keypad will send a signal correspond to the key pressed to the system

- Various of password is tried on the keypad and it passed correctly to the system

Stepper Motor

- As the User output at Check admin/user stage is high, a signal "1" will pass to the stepper motor and the stepper motor will start working

- Also, at the check input VHDL, once the button 1 is pressed.  The stepper will get a signal and then the stepper motor will also start working

Ram

- The Ram is being tested by checking its content directly.

- Once a password is entered, it retrieves data from the RAM

- In the password checking stage, the desired password is passed to the check password program to do a comparison.