EE 552

Advance Entry System

Simulation Documentation

Group:

Patrick Li 351018

Edgar Wong 356898

Edward Wong 263923

Howard Shum 338714

Professor: Dr. D. Elliott

Date: November 10,1998

# Table of Contents

Table of Contents	2

INTRODUCTION	3

Chip Usage	3

MODULE completed	3

Main Program--main_con.vhd	3

Keypad Decoder Module--keypad.vhd	3

Keypad BCD Encoder--dec_bcd.vhd	3

RAM--ram4X4.vhd	3

Ram read/write selector--rwselector.vhd	3

Message Initialization Module--message.vhd	3

User Message decoder Module--menu.vhd	4

Password verifier Module--passcom.vhd	4

Rom Module--rom1.vhd	4

Keypad Buffer Module--key.vhd	4

Keypad input to RAM Module--inpass.vhd	4

Task schedule	4

Diagram of design hierarchy	5

Appendix	6

Main Program	6

Keypad decoder Module	6

Keypad BCD encoder	6

Ran read/write selector	6

RAM	6

ROM	6

Message initialization Module	6

User Message decoder	6

Password Verifier	6

Keypad Buffer Module	6

Keypad input to RAM Module	6

# INTRODUCTION

This project includes the design and implementation of Controller. The controller is designed to control an electric motor, which attached to an advanced entry system. This controller provides user a option of scheduling the operation of the controller itself.  The design of the controller is mainly consisted of a keypad (input), LCD Display (Output) and the processing unit is design using the Altera chip. We designed the hardware by using Altera MaxPlus2 simulation. We also separated the functions into different subprograms in the sense of good programming and program reusing.

User can change and input password through keypad. All passwords will be store in RAM. Due to insufficient time, those optional functions canceled which mentioned in project proposal including Ethernet connection, optical transmission. We also encountered many unexpected problems during simulation, with a little bit change in VHDL code, the waveform will be a mess. We spent a lot of time in fixing this problem, which made us no time to do other optional functions.

# Chip Usage

We have used up ~45% with a speed of ~39MHz.

# MODULE completed

## Main Program--main_con.vhd

This VHDL code is used to link all subprograms together and perform the function ofAdvance Entry System. This part is huge, the waveform consists of all inputs and outputs, and it needs more than 10 pages to show all signals.

## Keypad Decoder Module--keypad.vhd

An interface for user to entry various functions of the controller. Since the keypad has 14-pin outputs, it needs 14 pins on the FPGA chip. This subprogram contains a delay function because of debouncing. The delay counter is 25175 since the clock speed is 25.175Mhz, it will give 1ms delay.

## Keypad BCD Encoder--dec_bcd.vhd

This program encodes keypad inputs to display on LCD with bcd encoding.

## RAM--ram4X4.vhd

The ram uses to storage of the schedule for the operating period on the controller and password to use the system. This system will use a 256K to store all data. It will take 16 I/O pins.

## Ram read/write selector--rwselector.vhd

This function is required in controlling read or write password from RAM and corresponding read/write vhdl code.

## Message Initialization Module--message.vhd

Initial display for user input password.

## User Message decoder Module--menu.vhd

Provide a friendly interface for user to set option on the controller, and all functions to administrator.

## Password verifier Module--passcom.vhd

After password has been input, it is needed to be verify. If the passdword is not right, no access will be allow.

## Rom Module--rom1.vhd

This is a read only memory, contains constant data.

## Keypad Buffer Module--key.vhd

This buffer is used to store intermediate input, and compare it with data in ROM.

## Keypad input to RAM Module--inpass.vhd

This VHDL code transfers all input signal directly to RAM.

# Task schedule

| Task | Start Date | Proposed completion date | Person | Duration | Completion |
| --- | --- | --- | --- | --- | --- |
| Master program | October 31 | November 7 | Edward | 10hrs | completed |
| Keypad BCD VHDL | October 15 | October 25 | Patrick | 4hrs | completed |
| Password verifier | October 15 | October 25 | Edgar | 3hrs | completed |
| LCD display connection VHDL | October 15 | October 25 | Edward | 3hrs | completed |
| LCD menu VHDL | October 31 | November 6 | Howard | 3hrs | completed |
| LCD initialization VHDL | October 15 | October 25 | Edward | 2hrs | completed |
| ROM Vhdl | October 31 | November 6 | Patrick | 5hrs | completed |
| RAM VHDL | October 31 | November 6 | Edgar | 3hrs | completed |
| Get password VHDL | October 25 | November 1 | Edward | 2hrs | completed |
| Check password VHDL | October 25 | November 1 | Edgar | 2hrs | completed |
| RAM-password VHDL | October 25 | November 1 | Patrick | 2hrs | completed |
| Change password VHDL | October 25 | November 1 | Howard | 2hrs | completed |
| Store-time (in-out record) VHDL | October 31 | November 7 | Howard | 2hrs | completed |
| LCD connection | October 31 | November 7 | Howard | 1hrs | Pending |
| Keypad buffer VHDL | October 31 | November 7 | Patrick | 2hrs | completed |
| Keypad connection | October 31 | November 7 | Howard | 1hrs | Pending |
| Keypad decoder VHDL | October 31 | November 7 | Patrick | 3hrs | completed |
| Code testing | October 31 | November 7 | Edgar/ Edward | 5hrs | Processing |

# Diagram of design hierarchy

main_con	LPM_ADD_SUB:361	addcore:adder

LPM_COMPARE:370	comptree:comparator	cmpchain:cmp_end

LPM_ADD_SUB:518	addcore:adder

LPM_COMPARE:527	comptree:comparator	cmpchain:cmp_end

LPM_ADD_SUB:750	addcore:adder

LPM_COMPARE:759	comptree:comparator	cmpchain:cmp_end

LPM_ADD_SUB:820	addcore:adder

LPM_COMPARE:829	comptree:comparator	cmpchain:cmp_end

Message:dis_message	LPM_ADD_SUB:111	addcore:adder

LPM_COMPARE:183	comptree:comparator	cmpchain:cmp_end

LPM_ADD_SUB:249	addcore:adder

LPM_COMPARE:274	comptree:comparator	cmpchain:cmp_end	comptree:comp	cmpchain:cmp3	cmpchain:cmp0	cmpchaim:cmp_end

Rom1:loadrom1	LPM_ROM:loadrom	altrom:srom

Key:key_buff	LPM_ADD_SUB:220	addcore:adder

LPM_ADD_SUB:532	addcore:adder

LPM_COMPARE:836	comptree:comparator	cmpchain:cmp_end	comptree:comp	cmpchain:cmp1

# Appendix

### Main Program

Code and Waveform

### Keypad decoder Module

Code and Waveform

### Keypad BCD encoder

Code and Waveform

### Ran read/write selector

Code and waveform

### RAM

Code and Waveform

### ROM

Code and Waveform

### Message initialization Module

Code and Waveform

### User Message decoder

Code and Waveform

### Password Verifier

Code and Waveform

### Keypad Buffer Module

Code and Waveform

### Keypad input to RAM Module

Code and Waveform