-------------------------------------------------------------

-- The Top-Level Entity

-- Architecture : structural

-- Name: toplevel2.vhd

-- Description  : This entity encompasses all of the input from the ADC

--		and the control path, the speech recognition module and the IR module

------------------------------------------------------------

library IEEE;

use IEEE.std_logic_1164.all;

library work;

use work.globals_pkg.all;

entity toplevel2 is

port (IN_CLOCK: in std_logic; --

rst_ex : in std_logic; --

pb_one, pb_two: in std_logic; --

SOC: 	out std_logic; --

EOC: 	in std_logic; --

ADC_in: 	in std_logic_vector(7 downto 0); --

ADC_clock: 	out	std_logic; --

LEDs: 	out	std_logic_vector(5 downto 1); --

IRinput :in std_logic; --

IRcode_out :out std_logic; --

left_sevseg : out std_logic_vector(6 downto 0); --

right_sevseg : out std_logic_vector(6 downto 0)  );  --

end entity toplevel2;

library IEEE;

use IEEE.std_logic_1164.all;

library work;

use work.globals_pkg.all;

package the_toplvl_pkg is

component clock_divider

generic(N : positive := 50);

port(clock, reset: in std_logic;

slow_clock : buffer std_logic  );

end component;

component DualSeven_decoder

port(	input_5bits: in std_logic_vector(4 downto 0);

out_leftsegment, out_rightsegment: out std_logic_vector(6 downto 0)

);

end component;

component control_in

generic	(	numwords	:	positive := 2;

sevsegwidth	:	positive := 5	);

port (	clock			: 	in 	std_logic;

reset			:	out	std_logic;	--global reset generated by the two PB's

pb_one, pb_two	: 	in std_logic;

LEDs			: 	out	std_logic_vector(5 downto 1);

sevsegSP		:	in	std_logic_vector(sevsegwidth-1 downto 0);

sevsegIR		:	in	std_logic_vector(sevsegwidth-1 downto 0);

sevsegdisp		:	out	std_logic_vector(sevsegwidth-1 downto 0);

sample_noise	:	out	std_logic;

sample_done		:	in	std_logic;

vtr				:	out	std_logic;

vt_done			:	in	std_logic;

word_recognized	:	in std_logic;

the_recog_word	:	in std_logic_vector(numwords-1 downto 0);

wack			:	out std_logic;

IR_pb1			: 	out std_logic;

IRt_done		:	in	std_logic;

IRcodeout		:	out	std_logic_vector(numwords-1 downto 0);

IRcontrolout	:	out	std_logic_vector(1 downto 0);

sack			:	in std_logic	);

end component;

component speech_process_small

port (SYS_CLK : in std_logic;  	-- input clock

RESET : in std_logic;  	-- active high reset

AD_DATA_IN : in std_logic_vector(a2d_width-1 downto 0);

AD_EOC : in std_logic;

AD_CLK : out std_logic;

AD_SOC : out std_logic;

NOISE_SAMPLE: in std_logic;  	-- controls noise sampling

SAMPLE_DONE: out std_logic;  	-- signifies noise sampling done

WORD_MATCH : out std_logic;  	-- indicates recognized word found

WORD_CODE : out std_logic_vector(word_width-1 downto 0);

TRAIN_DONE : out std_logic;  	-- indicates training complete

TRAIN_STATUS: buffer std_logic_vector(dispcode_width-1 downto 0);

VR_MODE : in std_logic );  	-- select between train and recog modes

end component;

component IRmodule

generic( commands : positive := 2;

count_width : positive := 12;

address_width : positive := 7;

part_width	: positive := 5;

dis_delay	: positive := 20;

trans_delay : positive := 10;

word_num : natural := 96);

port(	 clock, scroll, bit_stream, mode, switch2 :	in std_logic;

code_in : in std_logic_vector(1 downto 0);

code_out, trans_done_flag, train_done : out std_logic;

display : out std_logic_vector(4 downto 0)		);

end component;

end the_toplvl_pkg;

library work;

use work.the_toplvl_pkg.all;

architecture wholeshebang of toplevel2 is

signal mode_select,rst_int : std_logic;

signal newmode : std_logic_vector(1 downto 0);

signal clock,NEW_WORD1,WORD_VALID1 : std_logic;

signal noise_sample,noise_sample_done,vr_mode : std_logic;

signal train_status : std_logic_vector(dispcode_width-1 downto 0);

signal one,zero,nulls : std_logic;

signal SP_tdone,WORD_MATCH : std_logic;

signal IR_tdone,transmit_done,IR_pb1 : std_logic;

signal sevsegIR, sevsegSP, sevseg_int : std_logic_vector(4 downto 0);

signal codeCP2IR, WORD_CODE,IRcontrol : std_logic_vector(1 downto 0);

begin

wearsthepants: component control_in

port map(clock => clock,	--

reset => rst_int,		--

pb_one => pb_one,		--

pb_two => pb_two,		--

LEDs => LEDs,			--

sample_noise => noise_sample, 	--

sample_done => noise_sample_done,	--

vtr => vr_mode,						--

vt_done => SP_tdone, -- change this	--

word_recognized => WORD_MATCH, 		--

the_recog_word => WORD_CODE,  		--

wack => nulls,   -- change this, its an output

sevsegSP => sevsegSP, --

sevsegIR => sevsegIR,--

sevsegdisp => sevseg_int, --

IR_pb1	=>	IR_pb1,	--

IRt_done => IR_tdone, --

IRcodeout => codeCP2IR,	--

IRcontrolout=> IRcontrol, --

sack => transmit_done	); --

talktome: component speech_process_small

port map(SYS_CLK => clock, -- input clock

RESET => rst_int,-- active high reset

AD_DATA_IN =>ADC_in,

AD_EOC => EOC,

AD_CLK => ADC_clock,

AD_SOC  => SOC,

NOISE_SAMPLE => noise_sample,	-- controls noise sampling

SAMPLE_DONE => noise_sample_done,	-- signifies noise sampling done

WORD_MATCH => WORD_MATCH,	-- indicates recognized word found

WORD_CODE => WORD_CODE, -- the code corresponding to the recognized word

TRAIN_DONE => SP_tdone,	-- indicates training complete

TRAIN_STATUS => sevsegSP, --

VR_MODE => vr_mode  	-- select between train and recog modes

);

somethingyoucantsee: component IRmodule

port map(	clock => clock, --

scroll => IR_pb1,--

bit_stream => IRinput, --

mode => IRcontrol(0), --

switch2 => IRcontrol(1), --

code_in =>codeCP2IR, --

code_out => IRcode_out, --

trans_done_flag => transmit_done, --

train_done => IR_tdone, --

display => sevsegIR 	);  --

arealcutup: component clock_divider

generic map( N => 3)

port map(clock => IN_CLOCK,

reset => rst_ex,

slow_clock => clock		);

whogetsseen: component DualSeven_decoder

port map(	input_5bits => sevseg_int,

out_leftsegment => left_sevseg,

out_rightsegment => right_sevseg      );

end wholeshebang;

-------------------------------------------------------------

-- The Clock Divider

-- File Name    : clock_divider.vhd

-- Architecture : structural

-- Description  : This is the clock divider that is made use of

--		in the structural assembly throughout.

-------------------------------------------------------------

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity clock_divider is

generic(N : positive := 50);

port(clock, reset: in std_logic;

slow_clock : buffer std_logic  );

end clock_divider;

architecture behaviour of clock_divider is

signal clk: std_logic;

begin

twice_period: process(clock, reset)

variable cnt: std_logic_vector(N-1 downto 0);

begin

if reset='1' then

cnt := (others => '0');

else

if rising_edge(clock) then

cnt := cnt + '1';

if cnt = N then

cnt := (others => '0');

slow_clock <= not slow_clock;

else

slow_clock <= '0';

end if;

end if;

end if;

end process;

end behaviour;

-------------------------------------------------------------

-- The Input and Control Path

-- Author       : Scott Medynski

-- Student ID   : 266846

-- Date         : November 27, 1999

-- File Name    : control_in.vhd

-- Architecture : structural

-- Description  : This entity encompasses all of the input from the ADC

--				  and the control path.

-------------------------------------------------------------

library IEEE;

use IEEE.std_logic_1164.all;

entity control_in is

generic	(	numwords	:	positive := 2;

sevsegwidth	:	positive := 5	);

port (	clock			: 	in 	std_logic;

reset			:	out	std_logic;	--global reset generated by the two PB's

-- The hardware input and output

pb_one, pb_two	: 	in std_logic;

LEDs			: 	out	std_logic_vector(5 downto 1);

sevsegSP		:	in	std_logic_vector(sevsegwidth-1 downto 0);

sevsegIR		:	in	std_logic_vector(sevsegwidth-1 downto 0);

sevsegdisp		:	out	std_logic_vector(sevsegwidth-1 downto 0);

-- The inputs and outputs to the internal chip

sample_noise	:	out	std_logic;

sample_done		:	in	std_logic;

vtr				:	out	std_logic;

vt_done			:	in	std_logic;

word_recognized	:	in std_logic;

the_recog_word	:	in std_logic_vector(numwords-1 downto 0);

wack			:	out std_logic;

IR_pb1			: 	out std_logic;

IRt_done		:	in	std_logic;

IRcodeout		:	out	std_logic_vector(numwords-1 downto 0);

IRcontrolout	:	out	std_logic_vector(1 downto 0);

sack			:	in std_logic

);

end entity control_in;

library IEEE;

use IEEE.std_logic_1164.all;

package the_inctrl_pkg is

component ctrl_path

generic	(	numwords 	:	positive := 2);

port (	clock			: 	in 	std_logic;

reset			:	in	std_logic;	--global reset generated by the two PB's

mode_select		:	in 	std_logic;

newmode			:	in 	std_logic_vector(1 downto 0);

sevseg_select	:	out	std_logic_vector(1 downto 0);

IRtrain_done	:	in	std_logic;

IRcodeout		:	out	std_logic_vector(numwords-1 downto 0); -- change this to match the real number of IR codes

IRcontrol		:	out	std_logic_vector(1 downto 0);

send_ack		:	in std_logic;

sample			:	out	std_logic;

sample_done		:	in	std_logic;

v_train_or_recog:	out	std_logic;

vtrain_done		:	in	std_logic;

word_recognized	: 	in std_logic;

the_recog_word	: 	in std_logic_vector(numwords-1 downto 0);

word_ack		: 	out std_logic;

LED_readout		: 	out	std_logic_vector(5 downto 1)	);

end component;

component input

generic	(	sevsegwidth :	positive := 5);

port (	clock			: 	in 	std_logic;

pb_onein, pb_two	: 	in std_logic;

pb1_db_in		: 	in std_logic;

pb1_db_out		: 	out std_logic;

reset			:	out	std_logic;

mode_select		:	out	std_logic;

newmode			:	out	std_logic_vector(1 downto 0);

sevseg_readout	:	out	std_logic_vector(sevsegwidth-1 downto 0)

);

end component;

end the_inctrl_pkg;

library work;

use work.the_inctrl_pkg.all;

architecture incontrol of control_in is

signal mode_select,rst_int : std_logic;

signal newmode,sevseg_select : std_logic_vector(1 downto 0);

signal sevsegCP 		: std_logic_vector(sevsegwidth-1 downto 0);

signal input_pb1,pb1db_out,high	: std_logic := '1';

begin

reset<=rst_int;

get_all_inputs: component input

generic map(sevsegwidth => 5)

port map( 	clock => clock,

reset => rst_int,

pb_onein => input_pb1,

pb1_db_in => pb_one,

pb1_db_out => pb1db_out, ---*********

pb_two => pb_two,

mode_select => mode_select,

newmode => newmode,

sevseg_readout => sevsegCP	);

control_it: component ctrl_path

generic map(numwords => 2)

port map( 	clock => clock,

reset => rst_int,

mode_select => mode_select,

newmode => newmode,

sevseg_select => sevseg_select,

sample => sample_noise,

sample_done => sample_done,

v_train_or_recog => vtr,

vtrain_done	=> vt_done,

word_recognized => word_recognized,

the_recog_word => the_recog_word,

word_ack => wack,

IRtrain_done => IRt_done,

IRcodeout => IRcodeout,

IRcontrol => IRcontrolout,

send_ack => sack,

LED_readout => LEDs		);

-------------------------------------------------------------

-------------------------------------------------------------

-- The Seven Segment Display and PB1 Controller

thesevsegs:	process(clock)

begin

-- Now assign the proper value to sev segs according to the select signal

if rising_edge(clock) then

case sevseg_select is

when "00" =>  -- control path

sevsegdisp <= sevsegCP;

input_pb1 <= pb1db_out;

IR_pb1 <= high; -- tie it high because the signal is active low

when "01" =>  -- speech module

sevsegdisp <= sevsegSP;

input_pb1 <= pb1db_out;

IR_pb1 <= high;

when "10" =>  -- IR module

sevsegdisp <= sevsegIR;

input_pb1 <= high;

IR_pb1 <= pb1db_out;

when others =>

sevsegdisp <= sevsegCP;

input_pb1 <= pb1db_out;

IR_pb1 <= high;

end case;

end if;

high <= rst_int or not rst_int;

end process;

end incontrol;

-------------------------------------------------------------

-- The Control Path

-- Author       : Scott Medynski

-- Student ID   : 266846

-- Date         : October 26, 1999

-- File Name    : control_path.vhd

-- Architecture : behavioral

-- Description  : This is the modified small control path.

--

-------------------------------------------------------------

library IEEE;

use IEEE.std_logic_1164.all;

entity ctrl_path is

generic	(	numwords :	positive := 2);

port (	clock		: 	in 	std_logic;

--

--			INTERFACE TO THE INPUT LOGIC

-- for the input interface

reset		:	in	std_logic;	--global reset generated by the two PB's

mode_select	:	in 	std_logic;

newmode		:	in 	std_logic_vector(1 downto 0);

sevseg_select	:	out	std_logic_vector(1 downto 0);

--

--			INTERFACE TO WORD BOUNDARY

-- used by the initial_ctrl_path

sample		:	out	std_logic;

sample_done	:	in	std_logic;

-- for the interface to the voice training

v_train_or_recog	:	out	std_logic;

vtrain_done	:	in	std_logic;

--			INTERFACE TO VOICE RECOGNITION MODULE

--			used by the active control path

word_recognized	: in std_logic;

the_recog_word	: in std_logic_vector(numwords-1 downto 0);

word_ack		: out std_logic;

--

--			INTERFACE TO IR MODULE

-- used by the initial_ctrl_path

IRtrain_done	:	in	std_logic;

-- used by the active control path

IRcodeout		:	out	std_logic_vector(numwords-1 downto 0); -- change this to match the real number of IR codes

send_ack		:	in std_logic;

IRcontrol		:	out std_logic_vector(1 downto 0);

--

--			CONTROL OF THE 7seg DISPLAY

-- for the control of the 7 segment display output

LED_readout		: 	out	std_logic_vector(5 downto 1)

);

end ctrl_path;

library IEEE;

use IEEE.std_logic_1164.all;

use work.globals_pkg.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

architecture modern_major_general of ctrl_path is

-- Name states meaningful names.

type intstate_type is (initial, active,

noise_sample, noise_sampled,

vtrain, vtrained,

IRtrain, IRtrained	);

type actstate_type is (init, pwr_int, power,

chanup, chandown, surf_int, chansurf, chansurfing	);

signal intstate, next_intstate : intstate_type := initial;

signal actstate, next_actstate : actstate_type := init;

signal count2five, counted5, activate : std_logic:= '0';

signal beendone, CPmode: std_logic_vector(2 downto 0):= "000";

begin

--**********************************

-- This is the initial control path.

--**********************************

initial_state_logic : process(intstate, newmode)

begin

case intstate is

-- inital state

when initial =>

CPmode <= "000";

activate<='0';

v_train_or_recog<='1';	--train

if mode_select='1' then

if newmode="00" then

next_intstate<=noise_sample;

elsif newmode="01" then

if beendone(0)='1' then

next_intstate<=vtrain;

else

next_intstate<=initial;

end if;

elsif newmode="10" then

next_intstate<=IRtrain;

elsif newmode="11" then

if beendone="111" then

next_intstate<=active;

else

next_intstate<=initial;

end if;

else

next_intstate<=initial;

end if;

else

next_intstate<=initial;

end if;

when noise_sample =>

CPmode <= "001";

sample<='1';

if sample_done ='1' then

next_intstate<=noise_sampled;

else

next_intstate<=noise_sample;

end if;

when noise_sampled =>

sample<='0';

beendone(0)<='1';	-- set flag for noise sample done

next_intstate<=initial;

when vtrain =>

CPmode <= "010";

v_train_or_recog<='1';

if vtrain_done ='1' then

next_intstate<=vtrained;

else

next_intstate<=vtrain;

end if;

when vtrained =>

beendone(1)<='1';	-- set flag for voice training done

next_intstate<=initial;

when IRtrain =>

CPmode <= "011";

if IRtrain_done ='1' then

next_intstate<=IRtrained;

else

next_intstate<=IRtrain;

end if;

when IRtrained =>

beendone(2)<='1';	-- set flag for IR training done

next_intstate<=initial;

when active =>

activate<='1'; -- activate the active control path

CPmode <= "110";

v_train_or_recog<='0';

next_intstate<=active; -- for now...

end case;

end process initial_state_logic;

--**********************************

--**********************************

-- This is the active control path.

--**********************************

--**********************************

active_state_logic : process(actstate,word_recognized,counted5)

begin

case actstate is

when init =>

--		subset<=directwords; -- channel and power, specifically

IRcodeout <= "00"; -- just to stop oscillations

word_ack<='0';

if activate='1' then

if word_recognized='1' then

if the_recog_word = w_power then

next_actstate <= pwr_int;

elsif the_recog_word = c_up then

next_actstate <= chanup;

elsif the_recog_word = c_down then

next_actstate <= chandown;

elsif the_recog_word = w_surf then

next_actstate <= surf_int;

else

next_actstate <= init;

end if;

else

next_actstate <= init;

end if;

end if;

when pwr_int =>

word_ack <='1';

next_actstate <= power;

when power => --pwron/off

word_ack <='1';

IRcodeout <= w_power;

if send_ack = '1' then

next_actstate <= init;

else

next_actstate <= power;

end if;

when chanup =>

word_ack <='1';

IRcodeout <= c_up; -- same as w_on;

if send_ack = '1' then

next_actstate <= init;

else

next_actstate <= chanup;

end if;

when chandown =>

word_ack <='1';

IRcodeout <= c_down; -- same as w_off;

if send_ack = '1' then

next_actstate <= init;

else

next_actstate <= chandown;

end if;

--********************************************

--************ SURF AREA *********************

--********************************************

when surf_int =>

word_ack <='1';

IRcodeout <= "00";

next_actstate <= chansurf;

when chansurf =>

--		subset <= chanwords;

word_ack <='0';

IRcodeout <= c_up;

count2five <= '1';

if (word_recognized ='1') and (the_recog_word = w_surf) then

next_actstate <= init; --chan_int;

elsif send_ack='1' then

next_actstate <= chansurfing;

else

next_actstate <= chansurf;

end if;

when chansurfing =>

word_ack <='0';

count2five <= '0';

if counted5 = '1' then

next_actstate <= chansurf;

else

next_actstate <= chansurfing;

end if;

end case;

end process active_state_logic;

--**********************************

-- This is the delay counter for the surf function.

--**********************************

surf_delay :process(clock, count2five)

variable cnt : std_logic_vector(22 downto 0);

begin

if count2five='1' then

cnt := "00000000000000000000000";

counted5 <= '0';

else

if rising_edge(clock) then

cnt := cnt + '1';

if cnt = "11111111111111111111111" then

cnt := "00000000000000000000000";

counted5 <= '1';

end if;

end if;

end if;

end process surf_delay;

--**********************************

-- This is the state register for the initial control path.

--**********************************

intstate_register :process(reset, clock)

begin

if reset='1' then --reset the state to "initial"

intstate <= initial;

elsif rising_edge(clock) then -- advance to the next state

intstate <= next_intstate;

end if;

end process intstate_register;

--**********************************

-- This is the state register for the active control path.

--**********************************

actstate_register :process(reset, clock)

begin

if (reset = '1') or (activate='0') then

actstate <= init; --waitfor_act;

elsif rising_edge(clock) then -- advance to the next state

actstate <= next_actstate;

end if;

end process actstate_register;

--**********************************

-- This is the signal control for the IR module

--**********************************

IRsignals: process(clock, intstate, actstate)

begin

if not(intstate=active) then

if intstate=IRtrain then--or intstate=IRtrained then

IRcontrol<="11";

else -- for all other states in the initial control path, turn it off

IRcontrol<="00";

end if;

else -- the initial control path is set to active mode

if actstate=power then

IRcontrol<="10";

elsif actstate=chanup then

IRcontrol<="10";

elsif actstate=chandown then

IRcontrol<="10";

elsif actstate=chansurf then

IRcontrol<="10";

else -- for all other states in the active control path, turn it off

IRcontrol<="00";

end if;

end if;

end process IRsignals;

-- Now select the source of the seven segment displays

-- according to the actual mode of the CP

-- this is merely Karnaugh mapping

sevseg_select(1) <= (not CPmode(2) and not CPmode(1)) or (CPmode(2) and not CPmode(0));

sevseg_select(0) <= (not CPmode(2) and not CPmode(0)) or CPmode(1);

-- Now drive the LED readout according to the actual mode of the CP

-- this is merely Karnaugh mapping

LED_readout(1)<= '1';

LED_readout(2)<= (CPmode(1) and not CPmode(0)) or (not CPmode(2) and CPmode(0));

LED_readout(3)<= (CPmode(1) and not CPmode(0)) or (not CPmode(2) and CPmode(1));

LED_readout(4)<= (CPmode(2) and CPmode(1) and not CPmode(0)) or (not CPmode(2) and CPmode(1) and CPmode(0));

LED_readout(5)<= (CPmode(2) and CPmode(1) and not CPmode(0));

end modern_major_general;

-------------------------------------------------------------

-- The Input

-- Author       : Scott Medynski

-- Student ID   : 266846

-- Date         : October 30, 1999

-- File Name    : input.vhd

-- Architecture : structural

-- Description  : This is the stuff for pushbutton input and indicators of the control path’s status.

-------------------------------------------------------------

library IEEE;

use IEEE.std_logic_1164.all;

entity input is

generic	(	sevsegwidth :	positive := 5);

port (	clock			: 	in 	std_logic;

pb_onein, pb_two	: 	in std_logic;

pb1_db_in		: 	in std_logic;

pb1_db_out		: 	out std_logic;

reset			:	out	std_logic;

mode_select		:	out	std_logic;

newmode			:	out	std_logic_vector(1 downto 0);

sevseg_readout	:	out	std_logic_vector(sevsegwidth-1 downto 0)

);

end entity input;

library IEEE;

use IEEE.std_logic_1164.all;

package the_input_pkg is

component debounce

port(	clock: in std_logic;

keyin: in std_logic;

keyout: out std_logic

);

end component;

component inlogic

port(	clock		: 	in 	std_logic;

PB1			: 	in std_logic;	-- the mode scroll button

PB2			: 	in std_logic;	-- the mode select button

reset		:	out	std_logic;

newmode		:	out	std_logic_vector(1 downto 0);

mode_select	:	out	std_logic

);

end component;

end the_input_pkg;

library work;

use work.the_input_pkg.all;

architecture someinput of input is

signal rst_int, pb_one_db, pb_two_db	: std_logic;

signal sevseg	: std_logic_vector(sevsegwidth-1 downto 0);

signal zerotothree	: std_logic_vector(1 downto 0);

begin

reset <= rst_int;

sevseg_readout <= not('0'&'0'&'0'&zerotothree);

newmode<= zerotothree;

get_the_inputs: component inlogic

port map( 	clock => clock,

reset => rst_int,

PB1 => pb_onein,

PB2 => pb_two_db,

newmode => zerotothree,

mode_select => mode_select	);

debouncePB1: component debounce

port map( 	clock => clock,

keyin => pb1_db_in,

keyout => pb1_db_out

);

debouncePB2: component debounce

port map( 	clock => clock,

keyin => pb_two,

keyout => pb_two_db

);

end someinput;

-------------------------------------------------------------

-- The Input Logic and State Display

-- Author       : Scott Medynski

-- Student ID   : 266846

-- Date         : October 30, 1999

-- File Name    : inlogic.vhd

-- Architecture : behavioral

-- Description  : This is the input logic and state display.

-------------------------------------------------------------

library IEEE;

use IEEE.std_logic_1164.all;

entity inlogic is

port (	clock		: 	in 	std_logic;

PB1			: 	in std_logic;	-- the mode scroll button

PB2			: 	in std_logic;	-- the mode select button

reset		:	out	std_logic;

newmode		:	out	std_logic_vector(1 downto 0);

mode_select	:	out	std_logic

);

end entity inlogic;

architecture embedded of inlogic is

signal	count	: integer range 0 to 3;

begin

reset <= not(PB1 or PB2);

mode_select<= (not PB2) and PB1;

incr_count: process(PB1)

begin

if FALLING_EDGE(PB1) then

if PB2='1' then

count <= count + 1;

else

count<=0;

end if;

else

count <= count;

end if;

end process incr_count;

thenewmode: process(clock)

begin

-- Now assign the proper value to newmode according to the count value

if rising_edge(clock) then

case count is

when 1 =>

newmode <= "01";

when 2 =>

newmode <= "10";

when 3 =>

newmode <= "11";

when others =>

newmode <= "00";

end case;

end if;

end process;

end embedded;

-------------------------------------------------------------

-- The Push Button Debouncer

-- File Name    : debounce.vhd

-- Architecture : behavioral

-- Description  : This is taken from the student application notes as referenced in [1].

-------------------------------------------------------------

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity debounce is

port(clock: in std_logic;

keyin: in std_logic;

keyout: out std_logic

);

end debounce;

architecture behaviour of debounce is

signal clk: std_logic;

begin

milli_s_period: process(clock)

variable cnt: std_logic_vector (14 downto 0);

begin

if rising_edge(clock) then

cnt := cnt + '1';

if cnt = "110001001010111" then

cnt := "000000000000000";

clk <= '1';

else

clk <= '0';

end if;

end if;

end process;

debounce: process(clk, clock)

begin

if rising_edge(clock) then

if clk='1' then

keyout <= keyin;

end if;

end if;

end process;

end behaviour;