The Dynamic Picture Frame

Final Report

Dave Ng

dng@ualberta.ca

Emy Egbogah

eegbogah@ualberta.com

Melodie Lumague

mlumague@ualberta.ca

Nitin Parimi

nparimi@ualberta.com

Ben Talbot

ben@v-wave.com

# Declaration of Original Content

The project and the contents of this report are entirely the original work of the authors except as listed in the REFERENCE section and as follows:

- Push Button synchronizer(button_input_synchronizer.vhd) based on traffic light synchronizer, reference [13] ,

- Clock Divider based on Traffic Light synchronizer, reference[13],

- Debouncer incorporated into Push Button synchronizer based on 1999_w application note, reference[17]

- RS-232 input module modified/fixed from RS-232 application note, reference [6]

- JPEG decompression software complied into Nios, reference [10]

- SDRAM memory controller/Memory Module based on Hulk design, reference [29]

- IR circuitry based on IS1U60 datasheets, reference [7]

- VDAC external circuitry based on the ADV7120 datasheet, reference[4]

- External RS232 circuitry based on website design, reference[19]

- SVGA display module based on work from VGA application note, reference[24]

- Buffering module based on pipelined design given in class/class notes, reference[13]

- All comparators, shift registers and FF elements in the command module were generated with Altera’s Megafunction plug-in and edited for specific uses, reference[18]

- External Time Clock based on 24 Hour  Clock , reference [20]

- Sprite modules based on Probe Display Project, reference [21]

- Testbench file I/O based on Dr. Elliott’s EE552 class notes [30]

- Signed:

- Ben Talbot							DATE

- Emy Egbogah						DATE

- David Ng							DATE

- Melodie Lumague						DATE

- Nitin Parimi							DATE

# Abstract

The goals of this project were to design and assemble a Dynamic Picture Frame (DPF). The DPF device, developed in hardware using VHDL, was to be loaded on an Altera 20KE200 FPGA mounted on an Excalibur board. It allows a user to upload 24-bit color images from a serial RS-232 or RF interface and display them on an SVGA monitor with a number of additional features including a clock that displays the current time, and the ability to scroll through the pictures in memory.

The DPF implementation is divided into 4 subsystems: memory, control, display, and load. The following document describes the function of these subsystems and the interaction of the different components in the DPF. It also outlines the characterization of input/output sources and the resource requirements of the entire system.

The control, display and load modules were coded, compiled and simulated successfully.  However, problems were encountered when the different components were integrated with the memory module.  Due to time constraints, the SDRAM memory interface was eventually cut and exchanged for an LPM_RAM module.  The simulations and working code for the project can be found at the end of the report.

Dynamic Picture Frame

Data Sheet

Features:

- Image Uploading over a Wireless RS-232 link

- Real Time 12:00 clock that can be imposed overtop of an image

- IR control of picture system

- Automated picture Rotation

- 24bit Color @ 1024 X 768 Resolution Picture

SVGA Video Parameters

| Horizontal |  | Vertical |  |
| --- | --- | --- | --- |
| Resolution | 1024 | Resolution | 768 |
| Margins |  | Margins |  |
| Front Porch | 56 | Front Porch | 1 |
| Synch | 104 | Synch | 3 |
| Back Porch | 220 | Back Porch | 24 |

PinOuts

| System Pin | Type | Number of Pins | Description |
| --- | --- | --- | --- |
| - Clock | Input | 1 | Global 33Mhz Clock |
| - Reset | Input | 1 | Active Low Reset |
| # Load Module |  |  |  |
| - cmpserialin | Input | 1 | Serial Input line for RS-232 data |
| # Memory Module |  |  |  |
| - WEbar | Output | 1 | Write Enable Line |
| - CKE | Output | 2 | Clock Enable |
| - CSbar | Output | 1 | Chip Enable 1 |
| - CS2bar | Output | 1 | Chip Enable 2 |
| - addr | Output | 13 | Address Lines |
| - data | Output | 64 | Data Lines |
| - RASbar | Output | 1 | RAS select line |
| - CASbar | Output | 1 | CAS select line |
| - DQM | Output | 8 | Data Masking lines |
| - BA | Output | 2 | Bank Address Lines |
| # Display Module |  |  |  |
| - Blue | Output | 8 | Blue Color lines |
| - Green | Output | 8 | Green Color lines |
| - Red | Output | 8 | Red Color lines |
| - Hsynch | Output | 1 | Horizontal Synch |
| - Vsynch | Output | 1 | Vertical Synch |
| - Blank | Output | 1 | Blanking signal, clears the screen |
| - SVGA Clock | Output | 1 | SVGA 66Mhz Clock |
| - Ground | Output | 1 | Enables System Ports |
| # Command Module |  |  |  |
| - IRbitstream | Input | 1 | Serial IR line for IR data packet |

# System Resources

| SVGA Speed | 66 MHz |
| --- | --- |
| DPF Speed | 33 MHz |
| Memory Speed | 50 MHz |
| FPGA Logic Elements | 2238 |
| FPGA ESB Blocks | 103220 |

*- Maximum speed of the chip can not be determined using the PLL since returns numbers relative to multiple clock

###### Table of Contents

Declaration of Original Content	1

Abstract	3

Load Module	4

Memory Module	4

Display Module	5

Command Module	5

System Resources	5

The Project	8

Achievements	8

Operation and Implementation	9

Component Specification	9

Control Unit	9

Memory Module	9

Command Module	9

Display Module	9

SDRAM Controller	9

Load Controller	10

Subsystem Specification	10

Memory Subsystem	10

Control Subsystem	12

Display Subsystem	15

Decoder Subsystem	18

Load Subsystem	19

Signal Descriptions	20

Hardware Specifications	22

FPGA Size Measurement/Estimation	22

Command Module	22

Display Module	22

Load Module	22

Memory Module	22

Control Module (Elements were compiled on their own but removed)	23

Microprocessor Module (Removed)	23

Total  Logic Element(Without Removed Elements)	2238/8320(29%)	23

Testing and Characterization	24

SVGA and VDAC	24

Universal Remote Encoding Scheme	25

Problems with the SDRAM controller	26

References	27

Test Benches	29

# The Project

The Dynamic Picture Frame (DPF) is an electronic device that allows the user to download multiple images, which can be taken from the computer using an RF or RS-232 port. The main control of the DPF is through an IR remote, so that the user can quickly and easily change pictures at the push of a button. A simple menu interface allows any user to select images from an album stored in memory to create a small slide show, automatically scrolling through pictures or displaying a single, static image. A clock that displays the current time and can be manually set is also included.

# Achievements

The following is a list of stages that have been successfully implemented on the DPF:

- Output of uncompressed images 24-bit SVGA (1024x768) to monitor.

- Clock display with user-adjustable time.

- Serial cable (RS-232) interface which allows PC to memory image transfer.

- RF interface to allow wireless PC to memory image transfer.

- Capability to scroll through pictures on the DPF.

- Infrared control of DPF.

- Automated picture rotation.

Some intended components of the DPF were completed but could not be incorporated into the final design due to timing requirements; they are listed below:

- 8-word burst controller from SDRAM controller

- Full control system for handling loading/decoding and displaying

The following list was not completed due to time limitations

- Implementation of various stages of JPEG decompression.

- Menu interface for displaying images currently stored in memory.

- Higher Bandwidth memory interface

- USB/IDE interface.

# Operation and Implementation

In order to simplify the design and testing process, the DPF implementation was divided into a number of components. The components and the communication/data signals between them are shown in the module interaction diagram on the next page.  Each component is part of a specific subsystem that performs a single task on the DPF.

## Component Specification

### Control Unit

The control unit is the ‘brain’ of the system; it controls and oversees all operations of the DPF. It receives user instructions from the command module, and determines when the microprocessor and display module perform their decoding and SVGA outputting duties. The control unit has a strong interdependence with the memory module.

### Memory Module

The memory module controls any aspect of the DPF that accesses the external SDRAM. It handles all memory addressing, file organization, and data reading and writing. The control unit sends commands to the SDRAM controller including the index of which image to retrieve from memory and whether the image is to be sent to the display module. The memory module also communicates directly with the load controller to transfer incoming images to the SDRAM.

### Command Module

The command module contains a keypad decoder unit and an IR decoder unit. Each unit contains its own synchronizers that receive input from the keypad (pushbuttons) or the IR receiver. The command module deciphers these input signals and sends the corresponding user instruction to the control unit.

### Display Module

As its name implies, the display module is responsible for streaming the uncompressed image data to the VDAC for display on the SVGA monitor. Once it receives an enabling signal from the control unit, it will receive uncompressed image data from the memory module.  This data is combined with any necessary sprites, including the clock and the loading/decoding signals before it forwards the information to the VDAC.

### SDRAM Controller

The SDR (single data rate) SDRAM controller provides a simple interface to the SDRAM memory. It addresses the memory, and provides 8 commands that the memory module can call, including read, write, and auto-refresh.

### Load Controller

The load controller receives images that are to be stored in memory from the serial input controller which will decipher data from an RS-232 or RF signal. With proper timing signals, the load controller sends the images to the memory module. It acts independently of the control unit. The memory module informs the control module of the changed image count once the new images have been loaded.

## Subsystem Specification

### Memory Subsystem

The following is the theoretical description of the memory module, as it should ideally have worked:

The memory subsystem contains the SDRAM controller and the memory module. Memory in the SDRAM chip will be organized as shown in figure 1a.

Figure 1a: SDRAM Organization of the DPF

Starting at address 0, three 8MB blocks will be statically allocated for uncompressed image storage. Each block will begin with the uncompressed image size, which is followed by the uncompressed image data. Immediately following the 3 uncompressed image blocks is a statically defined table with 255 records, each containing a pointer to the starting location of a stored JPEG image, and the size of the compressed image. The first JPEG image starts immediately after the table of records and each subsequent image is stored immediately following the previous one. The size entry allows for variable-sized images to be stored without any space inefficiency or maximum image size limitations. The memory module will contain a number of registers which will contain the total number of JPEG images in memory, as well as the table offsets for each of the 3 uncompressed images in memory.

When a JPEG image is to be loaded, the memory module checks the number of JPEGs currently in memory, adds the image size and starting location to the next unused table record, loads the file to that starting location and increments the number of pictures in memory.

When an image is to be deleted, the image record is found and all the records below it are shifted up to replace it. This essentially invalidates the JPEG data in that block.

The indexes of the uncompressed images in memory are also updated as required. This simple deletion process creates segmentation by leaving holes of deleted images. In order to reduce overhead time, a defragmentation routine will only be called when a newly loaded image is within 3 MB of the maximum limit of the SDRAM chip. When this occurs, all images will be shifted up to fill the holes maintaining their order, and the record table will be updated.

Since almost every other subsystem is sending or receiving data from the memory subsystem, it will be designed with a multiplexor to arbitrate between requests for the data bus. Priority will be given to the display requests first, followed by the decoder, and then the load subsystem.

The preliminary design for the arbitrator handles requests based on a priority scheme, with the highest priority given to the Load Module, followed by the Microprocessor Module and then the Display Module. This design is preliminary until the speed of the memory module has been completely implemented and tested. This design was chosen because the Display Module can be fed from the sprite module while loading and decoding; therefore it will always have an image to display. Highest priority was given to the load module since it does not have the ability to communicate back to the PC to stop image transmission, and can only buffer the image before transmitting it to the memory module. Also, because the Microprocessor module includes interrupt processing ability, it can easily be interrupted during the decoding without losing meaningful work, thereby handling loads without losing data or displaying a blank screen.

To help relieve the congestion in the memory module, the SDRAM controller will transmit 8-word burst data. Therefore, all of the modules receiving input from the memory module will contain pipelined-buffers that allow the data to be stored without immediately processing it.

This module did not work when instantiated with the SDRAM memory control. As testing continued the difficulties with the memory module and haphazard changes caused many pieces of the code to be lost and left unimplemented.  Therefore, when the SDRAM memory interface was exchanged for an LPM_RAM module the control mechanism was reduced, since the RAM was now only required to alternate between 2 pictures.  The LPM_RAM module used to replace the SDRAM module made the interface very simple, as addresses are placed on the LPM_RAM address line with a read enable. The corresponding memory element is returned on the next clock cycle. The LPM_RAM used a dual-port to allow read and write accesses at the same time.

In order for our system to meet the bandwidth and storage requirements of the system, the SDRAM controller was modified to complete 8-word burst, along with increasing the address scheme of the system. When other modules, described in the theoretical description below, were added to the system unusual errors started occurring.  These errors included instantiating an unconnected module causing the entire memory system to begin reading incorrect values. Due to time and workload restrictions, these problems were not fully explored and were thus shelved for LPM_RAM implementation.

### Control Subsystem

The control subsystem, composed of the control module and the command module, provided the interface between the user input and the DPF activity. The command module received synchronized signals from the keypad and IR decoders which interpreted user signals from the keyboard and remote control. Giving priority to inputs coming from the keyboard decoder, the command module was to take the incoming signals and send corresponding command codes to the control module.

#### The Control Module

The control module was implemented with a state machine that controlled the functions of the display and decoder subsystems depending on the command code it received from the command module.  Each instruction, corresponding to a different button on the remote control, is listed in the table below:

| ##### Instruction | Description |
| --- | --- |
| On / Off | - Turns the DPF display on or off |
| Next | - Displays the next picture; - Scrolls up through minutes or hours to set time; - Menu control (optional) |
| Previous | - Displays the previous picture; - Scrolls down through minutes or hours to set time; - Menu control (optional) |
| Clock On / Off | - Turns the clock display on or off |
| Autoscroll On / Off | - Enables or disables automatic rotation of all images in memory |
| Set Clock | - Allows adjustment of the time displayed |
| Menu | - Brings up options menu (optional) |
| Enter | - Begins highlighted menu task (optional) |

Each instruction moved the state machine to a different state resulting in a different output instruction sent to the display module.  When the next picture is requested, the location of the displayed picture is incremented and the memory module is told to decode the new picture.  Upon receiving the decoding response from the memory module, an instruction is sent to the display module to display a “decoding” sprite until the decoding has finished.  A loading instruction from the memory module also caused the control module to change states, resulting in a “loading” sprite being displayed.  When the on/off button was pressed, it caused the display enable to be set either high or low; if the enable was set low, it caused the control module to move to the “off” state where it ignored all other inputs except those coming from the memory module, allowing decoding and loading to continue.

Other features that were implemented included a clock display and an autoscroll feature.  When the clock on/off button or autoscroll on/off was pressed, it caused either the external clock or autoscroll enable signal to the display module to be toggled.  The autoscroll feature was implemented using a counter that counted on a slower clock.  When the counter hit a certain value, it reset back to zero and the control module changed to the state that allowed the next picture to be displayed.   The DPF was designed so that when the next picture is called while the last picture in memory was displayed, it automatically scrolled to the first picture.

The control module was originally designed with the capability of supporting jpeg image decompression and a menu interface, which would have allowed the ability to store more, better quality images, as well as more user control; however, these features were cut due to time constraints.

#### The Command Module

As described briefly in the component specifications, the command module is responsible for taking user control input from both the IR stream and off button presses. Commands are routed to the control module along the instruction lines with a single enable pulse. If a collision occurs between IR command and keypad commands, precedence is given to the keypad commands.

##### Infrared (IR) Remote Control

The Digital Picture Frame used a remote controller to allow the user to scroll back and forth through all the pictures stored in RAM memory.  Therefore, an important part of the command module was to be able to accept IR packets from a remote controller and decode its instruction.  The first step in achieving this involved determining the coding scheme of the controller.  The X10 universal remote sent packets out using Pulse Width Modulation (PWM) but the exact coding scheme was dependent on the type of device the universal remote was configured to communicate with.  Secondly, the signal from the remote had to be decoded in order to determine which button on the remote was pressed.  Finally, once the IR receiver determined which button was pressed, the FPGA had to send out the appropriate signal so that the user request was correctly processed.

##### Encoding Scheme used by the Universal Remote Controller

There are 3 major packet construction types: NEC, MITSUBISHI, and SONY.  The NEC construction type was used because it was the simplest, with a 2-bit startup pattern, an 8-bit device ID and an 8-bit command. Although this packet construction is standard, the timing between the bit patterns changes depending on the device used. The final remote controller device chosen was a NEC VCR code 136. This scheme had a unique feature that some of the other NEC schemes did not include. It would only send the 2-bit startup code on the first packet if the buttons were held down; therefore it included a built-in debounce feature, which meant a single button press only resulted in one data packet transmission.  A detailed description of determining the encoding scheme can be found in the “Testing and Characterization” section.

##### Decoding the Received IR Pulse-Modulated Signal

The next task in using a remote controller was to decode the received pulse-modulated signal into the appropriate bits using the FPGA.  For example, the bit pattern of the ‘Play’ command is ‘1100001000110000’.  In order to recognize this pattern, a strategy for distinguishing ‘1’s from ‘0’s had to be implemented.  A top level diagram illustrating the strategy is shown and explained below:

The system above used a counter to time how long an input pulse remained in its “high” state.  The counter started timing when a rising edge was detected from the data packet.  The counter continued to count the number of clock cycles the signal was high until a falling edge was detected.  When a falling edge was detected, the value for “high time” of the pulse was stored in a clocked register, which was disabled until a falling edge was detected.  At this point, the counter was reset and ready to measure the duration of the next incoming pulse.  The D flip-flop was used so that the present value of the counter was sent to the register before the next rising edge enabled the counter.

The data (i.e. the number of clock cycles) was sent to three comparators to determine what type of bit had been received.  Each of the comparator units compared the incoming data to a range of predefined duration lengths that had been set to define the durations of the various bits.  When the nature of the bit was determined (i.e. 1, 0, or start bit), the bit entered a finite state machine (indicated on figure 2 as ‘fsm’).  The finite state machine detected the pattern of the bits in the data packet and as a result, detected what type of command had been sent from the remote control.  This instruction was then sent with an enable to the control module to implement the command.

##### Keypad Decoder

The keypad decoder decoded button presses from the external system push buttons. When a button is pressed its mapped command is placed on the instruction lines and sent to the command module. Only a single enable pulse is created for any length of button press. This is accomplished through the combination of button debouncing and state latching. This enable system is used throughout the DPF to ensure multiple commands are not issued for a single instruction.

### Display Subsystem

The display system is responsible for streaming the uncompressed image data to the VDAC for outputting to the SVGA monitor. The control unit instructs the memory module which uncompressed image is to be displayed, and this image is sent to the display module where it is combined with sprite data before being outputted to the VDAC. The display module will add clock and/or user message sprites to the image before sending it to the VDAC depending on the state which is determined by the control module. Pixel data will be buffered into the display module allowing the system to queue multiple pixels and allow the system to easily handle continual updating of the screen with new data.

#### The Display Module

The display module has two major functions: First, it must generate the proper blank, horizontal and vertical sync pulses to properly synchronize the display to the monitor. Figure 1b shows a diagram of the timing required to make these signals work.

Figure 1b: Timing requirements for synchronization signals

Horizontal and vertical sync pulses are carefully chosen to set the size, and margins of the display on the monitor. Although not shown in the diagram, the vertical sync signal also has front and back porch, and sync times, and they are usually counted in terms of the number of horizontal sync pulses. The DPF’s design uses these signals to generate a 1024x768 pixel displayable area, and an active low blanking signal was sent to the VDAC during the blanking intervals to hold the image data lines low.

The second function of the display module is to request pixel color information from memory and send these colors to the VDAC to generate the image on the screen. This required careful timing of memory requests as any problems in timing resulted in smearing of colors or distorted images. With the original SDRAM model, memory was automatically retrieved from memory using a series of fifos which converted accepted 64-bit words from memory and returned 24-bit pixel data every clock cycle. However, once the design was changed to support LPM_RAM, not enough pixel data was available to support 1024x768 images. Therefore, the display module was modified to display 52x39 images by expanding each color to 16x16 pixel blocks which required more complicated memory requests.

Once the proper image was displayed on the screen, sprites were overlaid on the display for the clock and loading image message. While loading, a car which “drives” across the screen is also displayed. The use of MIFs and sprites is discussed below:

##### Mif Files

To design display sprites, a Memory Interfacing File (MIF) is needed. Two MIFs are used for the design. The first file is a character definition file design by “Logic Analyzer” project team. The second file contains all the words to be displayed. To define the words to be used, an 8-bit starting address is defined, as shown below.

00000000 : 02 24 21 21 04 15 23 44 23 10 14 04 45; % CURRENT TIME: %

After referencing the appropriate characters for the word, the next 8-bit address is calculated by adding the length + 1 of the newly created word to the previous bit address.

(i.e length of current time is 13, so the new 8 bit address is 00001110).

##### Sprites

To define a sprite using the display module, two addressing signals, row_address and col_address, are created. Row_address and col_address  are defined by the 7 most significant bits of the vertical sync counter and horizontal sync counter of the display module, respectively. In addition, the rom_mux_output signal must be addressed in order to set the color of each pixel on-screen, row by row. If a pixel is in use, rom_mux_output is equal to 1 and the associated color is displayed. If the pixel is not in use, rom_mux_output is equal to 0 and the specified default color is used. To display a word, the signal format_address must be assigned the 8-bit address of the needed word in the mif.

Four main sprites were designed for the display module. The first sprite is for the display of the hour clock. We were unable to interface the display module with the control module, so we simulated the command to turn the clock on or off (clock_on) . When clock_on is asserted, the hour clock sprite is displayed and the clock is shown counting with the following format: XX:XX:XX PM. The signals row_address, col_address, and rom_mux_output must be considered to show the clock counting. The border drawn around the clock is defined by defining border color for the needed ranges of row_address and col_address. The second sprite is that of a car. The car is used to showcase the 24 bit color capabilities of the display module. Unlike other sprites defined for this sprite, the car has the ability to stream across the screen flashing different colors. To make this possible, a signal for each row of the car is defined. The signals are incremented on the incrementing of the seconds, providing a car animation.

The third and fourth sprites are used to display the current status of picture display. Two signals, loading_on and decoding_on are used to toggle the messages “Loading Image” and “Decoding Image”, respectively. When loading_on and decoding_on are deserted, no sprite will be displayed. On the other hand, when loading_on and decoding_on are asserted, the loading_on command will have precedence and only the “Loading Image” sprite will be displayed.

##### The Time Module

The 24 hour clock used for the DPF project is an enhancement of the 24 hour clock developed by the “Smart Alarm Clock” project. The additional enhancements are listed below.

 Ability to set the time

 Addition of seconds for precision timing

 Automatically set AM/PM

The clock for this project is 66MHZ. Since the hour clock runs on 2HZ, the system clock must be clock divided by 33 MHZ to achieve the needed slow clock.

To add seconds to the hour clock, two extra signals and a seconds counter variable were added. The two signals represent the first and second digits of a second (ie 12:00:XX).

The seconds counter is used to time the increment of the seconds. The counter is incremented on each slow clock until it reaches a value of 2 ( 2 samples/sec * 1 sec = 2 samples). When the counter is equal to 2, it is reset to zero and the seconds signals are incremented in the same manner as the minutes in the hour clock.

The addition of the ability to set the hour clock adds flexibility. The initial plan for setting the time was to process an instruction from the control module. If that instruction was for setting time, the hour clock would stop and the user would be able to set the time.

Unfortunately, as stated before, we were unable to interface the control module with the time module, so the set clock toggle was assigned to a dipswitch on the FPGA.  In addition, two push buttons were assigned to increment the minutes and hours. When the set clock switch is activated, the hour clock is stopped and the user is able to adjust the time using the push buttons. Each press of the push button increments the minutes/hours by several numbers because we were unable to find a perfect divisor for the button synchronizer debouncer. When the set clock is disabled, the hour clock resumes it’s counting.

### Decoder Subsystem

The decoder subsystem design involves communication between the microprocessor, the memory module, and the control unit. The design of the DPF is to have 3 uncompressed images in SDRAM: the currently being displayed, the next image, and the previous image. Therefore, when the next or previous image is requested by the user, it is already decompressed and ready for displaying. When the image being displayed changes, a new image is needed to maintain the 1 picture buffer, so managed by the control unit, the new JPEG image to be decoded is sent from the memory module to the microprocessor. The microprocessor decodes this image, and sends back an uncompressed version which is stored in the 8MB buffer.

Unfortunately, due to memory problems, and time restraints, this subsystem was removed from the final design, and the DPF now only supports the displaying of uncompressed images.

### Load Subsystem

The load subsystem uses the memory module and the load controller. The original design of this system was to be timed in such a way that downloading images could be done in the background, and would not interfere with the decoding or displaying processes. However, when the SDRAM was the planned memory device, the load module was changed to simplify the design and reduce the stress on the memory bandwidth. This was to be accomplished by having the load subsystem stall image displaying when any images were ready for uploading to the system. Once an image had been completely loaded into memory, the memory module incremented the number of images in the memory counter and added a record in the stored image table. With the LPM_RAM interface, the logic was simplified and therefore the load module could concurrently write to the unused memory location while the display module read from the second location to display its image.

The load module works by receiving 8-bit RS-232 packets from the serial connection and concatenating them into 24-bit words to be stored in RAM. The load module used a 24 word FIFO to handle any slack of extra packets that appear while the system cannot handle them. When the first packet arrives the system assumes that a complete picture is to be loaded and will stay in the loading state until it receives the full picture (based on size) or until a reset is issued to the system.

In order to send the packets over a connectionless interface, an RF interface was placed on top of the RS-232 interface using Abacom TX/RX-DFM-12V transmitter and receiver modules. After properly assembling the schematic circuits, this interface automatically transmitted RS-232 packets without any additional encoding. Initial tests proved to be quite noisy, but by adjusting the squelch level, a clean, accurate signal was consistently transmitted.

## Signal Descriptions

The following table briefly describes the purposes of each signal found in the module interaction diagram:

| Path (source to destination) | Signal | Width | Description |
| --- | --- | --- | --- |
| SDRAM controller (SD) to Memory Module | Data | 64 | Data out bus from SDRAM |
|  | Init_done | 1 | Initialization signal, used to inform the memory module that SDRAM module is finished initializing |
|  | tx_data, rx_data | 1 each | Transmit and receive signals for synchronizing the word request to the fifo |
|  | r_ack,w_ack | 1 each | Read and write acknowledgement command from the SDRAM controller |
| Memory Module to SDRAM controller | R_enable | 1 | Read enable signal to the SDRAM controller |
|  | W_enable | 1 | Write enable signal to the SDRAM controller |
|  | RW_address | 24 | Address line for requesting data |
|  | Data | 64 | Data bus for sending and receiving data |
|  | Data_mask |  | Data masks for controlling byte values |
| Memory Module to Microprocessor | M_data_in | 32 | Compressed image information that needs to be decompressed |
|  | M_valid_in | 1 | Validates data on M_data_in bus |
|  | M_data_out | 32 | Uncompressed image information that needs to be stored |
|  | M_valid_out | 1 | Validates data on M_data_out |
| Memory Module to Control Module | Total_pic | 8 | Tells controller how many total pictures are in the library. |
|  | Mem_response | 4 | Tells controller which commands to perform |
| Control Module to Memory Module | Curr_task_enable | 1 | Enables memory module to do current task |
|  | Display_loc | 2 | Tells memory mod the location of picture to be displayed |
|  | M_decode_loc | 8 | Location in table of current jpeg image being decompressed |
|  | Curr_task | 4 | Instruction lines to memory module |
|  | Decode_loc | 2 | Location that current microprocessor image will be place |
| Memory module to load controller | Clock | 1 | System clock |
|  | Reset | 1 | System reset |
| Load controller to memory module | Mem_load_word | 64 | data bus from controller to memory for new pictures |
|  | Word_request | 1 | Validates information on mem_data_in |
| Memory module to Memory FIFO | Fifo_wenable | 1 | Fifo write enable signal |
|  | Fifo_data_mem | 64 | Data input line into the FIFO |
|  | Fifo_used | 6 | Contains the current level of the fifo |
| FIFO module to Display module | Fifo_renable | 1 | Read enable signal into the fifo |
|  | Fifo_clear | 1 | Clears the fifo at the end of a display line |
|  | Fifo_empty | 1 | Tells the display module whether the FIFO is empty |
|  | Fifo_data_disp | 64 | Data output lines from the FIFO |
| Control Module to Display Module | Disp_instruct | 4 | Interface commands to clock |
|  | Disp_instruct_enable | 1 | Validates data in disp_instr bus |
|  | Display_on | 1 | Signal enabling the display to be on. |
|  | Ext_clk_enable | 1 | Enables external clock to display |
|  | Reset | 1 | System reset |
|  | Clock | 1 | System clock |
| Microprocessor to Control Module | M_done | 4 | Completion and status information about jpeg decompression |
| Control Module to Microprocessor | M_done_enable | 1 | Enables and validate information on M_done |
|  | M_go_enable | 1 | Enables and validates information on M_go_enable |
|  | M_go | 4 | Start and hold information of jpeg decompression |
| Control module to command module | Clock | 1 | System clock |
|  | Reset | 1 | System reset |
| Command module to Control module | Enable | 1 | Enables and validates data on instruction bus |
|  | Instruction | 4 | Input commands from IR and keypad interface |

# Hardware Specifications

## FPGA Size Measurement/Estimation

### Command Module

Keypad Decoder					133/8320 (2%)

PWM Decoder					103/8320 (1%)

Command Decoder					618/8320 (8%)

Total Logic Requirement				901/8320 (10%)

### Display Module

#### Completed Section

Clock 						100/8320 (1%)

Sprite						169/8320 (2%)

Screen Updating 				228/8320 (1%)

-Includes state machine for updating colors, and requesting memory

Sprite Synchronization			37/8320

5648/106496 ESB Blocks

- Incorporates Sprites into the display module

### Load Module

#### Completed Section

Serial Port Interface 				129/8320 (2%)

Data Buffering 				84/8320 (1%)

Buffer/Memory Synchronization		32/8320

- 24 word deep FIFO			768/106496 ESB Blocks

### Memory Module

#### LPM RAM

Memory interface  				523/8320

96804/106496 ESB Blocks

- includes simple control logic for 2 pictures

#### SDRAM Section (Elements were compiled on their own but removed)

SDRAM Interface 				350/8320 (4%)

Output Display Buffering 			102/8320 (2%)

- LPM-fifo component with buffering of 128 word depth

Arbitrator					42/8320

- Similar multiple stage machine design to the SDRAM interface although somewhat less complicated since the program doesn’t have to produce the periodic refreshes

### Control Module (Elements were compiled on their own but removed)

#### Simplistic Section

Simple control state machine			35/8320

#### Full Section

#### Command State Machine			92/8320 (4%)

### Microprocessor Module (Removed)

#### Estimation

Nios embedded processor 			3000/8320 (36%)

- Nios documentation estimates the processor at about 30% of the total logic gates on the system

SRAM/FLASH Control Logic		300/8320 (4%)

- Control logic into the sram and flash will have similar requirements as that of the sprite synchronization in the Command Module.

Nios SDRAM synchronization 		300/8320 (4%)

- data buffering and control logic with the memory module

### Total  Logic Element(Without Removed Elements)	2238/8320(29%)

Currently all designed/estimated packages will fit in the provided chip.

# Testing and Characterization

## SVGA and VDAC

An integral part of the project is the presentation of the image on the monitor. However, in order to get a picture on the screen, the proper color-levels and synchronization pulses had to be determined.

To properly synchronize data with a monitor, video information must be sent in pulses along with the corresponding horizontal and vertical synchronization pulses. To determine the timing of these pulses, the front and back porch, and sync values for VGA (640x480) synchronization were scaled to match SVGA resolution (typically 800x600, or 1024x768). However, this method either failed to synchronize with the monitor resulting in a blank screen, or displayed a distorted image on the screen. Even when these pulse lengths were slightly modified, the test image, of a solid block of red, failed to produce a stable picture. Finally, after a few hours of research, a calculator was found that evaluated the synchronization timing using the VESA general timing formula. (ftp://ftp.vesa.org/pub/GTF/GTF_V1R1.xls). Using these values, the monitor displayed a stable image.

Once the monitor properly displayed data, the voltage level of the red, blue, and green signals had to be modified to allow for different colors. The suggested connection diagram of the ADV7120 (the video digital-to-analog converter selected for this task) can be found in the Appendix. During the characterization of this device, it was found that changing Rset which is connected to the full-scale adjust (FS adjust) did very little to actually affect the range of a color, so it was maintained at the suggested 560. Simply attaching 1.235V to Vref resulted in a short, so a diode was placed to control the current direction. It was later found that Vref sunk current out of the chip, so to maintain the required voltage, two forward-biased diodes were placed in series ( 0.65V x 2 = 1.3V ) .

The final stage in characterizing the VDAC was to control the color intensities sent to the monitor. The ADV7120 generates each color (red, green, blue) from the digital inputs by varying the current levels sourcing out of the color output ports. Maintaining a range of approximately 0 to 0.7V without exceeding the maximum 1.1V level proved to be more complicated than a simple voltage division because the final current also depended on the output load. Once Vref was properly set, and a 75Ω resistor was placed in parallel with the 75Ω monitor, the output voltage gave a range of 0.03V – 0.72 V.

The various colors were first tested by manually changing the color input pins from high to low and observing that the screen appropriately adjusted the corresponding color intensities. A simple VHDL program was also written to change the entire screen to any of the 16777216 possible colors. This code can also be found in the Appendix (crt_test).

## Universal Remote Encoding Scheme

The pulse lengths, device IDs, and command sequences were measured by capturing packets on the oscilloscope.  The oscilloscope was hooked to the IS1U60 IR receiver chip to determine the encoding scheme of the transmitter.  The square wave pattern displayed on the scope was analyzed to determine the how each of the logic levels were represented.  In this particular encoding scheme, pulse-width modulation was utilized.  High and low bits were both represented by high pulses, but they were differentiated by the fact that pulses representing a logic high bit had a longer duration than those representing logic low bits.  Furthermore, this encoding scheme involved a start bit whose associated pulse duration was longer than both the other pulses.  Each data packet transmitted by the remote started with a start bit that was followed by a pulse pattern representing ‘1’s and ‘0’s.  Each button on the remote sent out a different data packet (indicated on an oscilloscope by a different pulse pattern).  An example of the square-wave pulse pattern resulting from the ‘Play’ button on the remote is shown in Figure 1.  The pulse lengths, device IDs, and command sequences are summarized in the tables below.

Table 1: Bit timing values

| Pulse Timing | Voltage Level | Bit Value |
| --- | --- | --- |
| 8.5 ms | Low | Start Bit 1 |
| 4.1ms | High | Stop Bit 2 |
| 1.5 ms | High | ‘1’ |
| 0.4 ms | High | ‘0’ |
| 24 ms | High | Inter-word gap |
| 0.6 ms | Low | Inter-bit gap |

Table 2: Device ID and command ID sequences

| Sequence Name (Type) | Bit Pattern (Hex) |
| --- | --- |
| NEC 136 (Device ID) | 0xC2 |
| PLAY (Command ID) | 0x30 |
| FF (Command ID) | 0x60 |
| REW (Command ID) | 0xE0 |
| PAUSE (Command ID) | 0xB0 |
| REC (Command ID) | 0x33 |
| STOP (Command ID) | 0xC0 |

## Problems with the SDRAM controller

While trying to implement the SDRAM, problems with both the timing requirements of the system and with the LPM_FIFO components arose. The first implementation of the Memory module buffered an entire line into the LPM_FIFO once the display module requested memory. Because the desired resolution was 800x600, this required a queue of approximately 256, assuming that no refreshes occurred during a burst. The LPM_FIFO components had problems when extending them to very large values (64 bits x 256 words), which resulted in timing problems and corrupt data. In order to alleviate this problem the memory module was redesigned to perform frame burst. This frame burst was 3 memory module bursts of 8 words. The frame module allowed the FIFO requirements to drop to a worst case of 24 words, which helped the timing and signal requirements of the FIFO. Although this removed some of the timing problems of the system, corrupt data was still being retrieved from SDRAM.  In order to simplify the memory module requirements even further the Memory module was resigned to perform 8 bit word requests. This relieved some of the pressure on the FIFO to produce words at a very fast clock rate. This solution provided limited success, and was used as a demonstration of the Memory module.

The 128MB RAM had different timing requirement than the 32MB RAM module, for which the original SDRAM controller was designed. In order to try and receive the optimal timing requirement, many parameters, such as Refresh Rate, were adjusted. Because the compile times on the system are close to 20 minutes this was a very time consuming task. Hence the entire SDRAM memory was dropped to accommodate the LPM_RAM and demo completion dates.

# References

- Altera. (2000, June). Excalibur Embedded Processor Solutions. Retrieved January 16, 2002, from WWW: http://www.altera.com/products/devices/excalibur/exc-index.html

- Altera. (2000, June). Excalibur Development Kit Data Sheet. Retrieved January 16, 2002, from WWW: http://www.altera.com/literature/ds/ds_exckit.pdf

- Altera. (2000, June). White Papers: Excalibur Backgrounder. Retrieved January 16, 2002, from WWW: http://www.altera.com/literature/wp/excal_bkgrnd.pdf

- Analog Devices. CMOS 80MHz, Triple 8-bit Video DAC ADV7120. Retrieved January 19, 2002, from WWW: http://www.analog.com/productSelection/pdf/adv7120.pdf

- Altera. SDRAM Controller. Retrieved February 1, 2002, from WWW: http://www.altera.com/products/ip/altera/m-alt-sdr-sdram.html

- Tim Bensler & Eric Chan. RS-232 Serial Port Retrieved February 1,2002, from WWW: http://www.ee.ualberta.ca/~elliott/ee552/studentAppNotes/1999_w/RS232/

- Sharp Electronics. IS1U60/IS1U60L Datasheet. Retrieved January 28, 2002, from WWW: http://www.ee.ualberta.ca/~ee401/datasheets/IS1U60.pdf

- Lane, T. (1995). comp.compression Frequently Asked Questions (part 2/3). Retrieved January 26,2002, from WWW: http://www.faqs.org/faqs/compression-faq/part2/

- Lane, T. (1995). JPEG image compression FAQ, part 2/2. Retrieved January 26,2002, from WWW:  http://www.faqs.org/faqs/jpeg-faq/part2/

- Lane, T. (1995). IJG Subroutine Library. Retrieved January 26,2002, from WWW:   http://www.ijg.org/files/

- ABACOM Technologies. TX-DFM-12V 19200 bps Digital FM Transmitter Module. Retrieved February 4, 2002, from WWW: http://www.abacom-tech.com/catalog/tx_dfm.pdf

- ABACOM Technologies. RX-DFM-3V3 Digital FM Superhet Receiver. Retrieved February 4, 2002, from WWW: http://www.abacom-tech.com/catalog/rx_dfm.pdf

- Dr. Elliott. Traffic light synchronizer. Page 72 EE552 notes.

- Altera. (2000, June). Excalibur Pinouts. Retrieved February 4, 2002, from WWW: http://www.altera.com/literature/dp/excalibur/epxa10.pdf

- Altera. (2000, June). Nios SODIMM Support. Retrieved February 2, 2002, from WWW: http://www.altera.com/literature/manual/mnl_nios.pdf

- Samsung (2002, Jan). DRAM datasheets. Retrieved February 5,2002, from WWW: http://www.samsungelectronics.com/semiconductors/search/datasheet.jsp?family=243

- . Feng Gui, Taozhi Peng & KhernYang Tkwang; Push Button Debouncer February 26,2002, from WWW:  http://www.ee.ualberta.ca/~elliott/ee552/studentAppNotes/1999_w/keypad/App.html

- Altera’s megafunction Generator; provided with Altera tools

- Wichit Sirichote; RS232 Level Converter; February 26,2002, from http://www.kmitl.ac.th/~kswichit/MAX232/MAX232.htm

- Albert Chan & Dan Sirois. 24 Hour Clock Retrieved March 1,2002, from

http://www.ee.ualberta.ca/~elliott/ee552/studentAppNotes/1998_w/smart_alarm_clock/ap_note2

- Hao Luan, Bo Liu,  & Albert Chan, Video Display App Note http://www.ee.ualberta.ca/~elliott/ee552/studentAppNotes/1998_w/dicerace_video_display/

- Steve Dillen; PWM decoding scheme; February 25, 2002. Talking with TA

- Kevin White; IR Remote specifications; February 18,2002, from http://www.coe.montana.edu/ee/cady/misclink.htm

- Tetris Team; 16 Color VGA; Winter 1999; http://www.ee.ualberta.ca/~elliott/ee552/studentAppNotes/1999_w/16Colors/

- Image Indexing Group; 64-Color VGA Display; Fall 2000; http://www.ee.ualberta.ca/~elliott/ee552/studentAppNotes/2000f/interfacing/64_color_display/

- VESA standards; www.vesa.org

- ViewSonic Specifications; viewsonic.com

- Engdahl, Tomi; VGA Timing Information; http://www.epanorama.net/documents/pc/vga_timing.html

- Benj Carson; Jeff Mrochuk; SDRAM controller.

- Dr. Elliott. Testbench examples. EE552 notes, 2002.

# Test Benches

Test bench waveforms were not included with the test bench code submitted because the validity of the test benches were confirmed by the presence of exceptions generated while running the test bench.

The first test bench tests the basic functionality of the command module. The table below describes each test from the attached vector file:

| Time | Irbit | Keypad | Enable | Instruction | Description |
| --- | --- | --- | --- | --- | --- |
| 2000 | 0 | 1110 | H | LLLH | Instruction to go to Next Page |
| 4000 | 0 | 1101 | H | LLHL | Instruction to go to Previous Page |
| 6000 | 0 | 1011 | H | LHLL | Instruction to go to Menu |
| 8000 | 0 | 0111 | H | HLLL | Clock Toggle (On/Off) |
| 10000 | 0 | 1111 | L | LLLL | No IR/Keypad instruction |
| 12000 | 1 | 1111 | H | XXXX | Instruction from IR |
| 14000 | 1 | 1011 | H | LHLL | Keypad input takes precedence over IR input |

The second test bench tests the basic functionality of the load module. More specifically, it tests the loading of a 64 bit word into a FIFO using the rs232 for input. Eight bytes are sent at a time to form the 64 bit word. The vector file is attached on the next page.