# Providing Digital Inputs With a PC

Sometimes it is necessary to manually provide digital inputs to a digital system.  One method is to use switches of some kind to supply the desired logic levels.  This is inconvenient as the number of inputs grows large or if a complex sequence of inputs is desired.  This could be accomplished with simulations using CAD tools but sometimes this is not possible.  This document describes a simple method for providing digital inputs with commonly available equipment - a PC parallel port.

Most PC's are equipped with one or more LPT ports.  These ports provide a number of inputs and outputs that could be interfaced to a digital system.  Although there are a total of 12 digital outputs and 5 digital inputs available on the 25 pin connector, this simple design uses only 8 of the outputs. With the addition of some hardware these 8 outputs can be expanded arbitrarily.  Many schemes are possible but this design uses 4 LS74164 serial to parallel shift registers to expand the 8 bits to a total of 32 user programmable bits.

Note: It is important to remember that the outputs of this circuit are parallel shift registers ; their outputs will fluctuate as data is loaded into them and should only be sampled after they have settled.

# Circuit

See … for a MaxPlus2 schematic showing how the circuit is connected.

See … for a the pin out of the LPT ports DB25 connector

See … for the LS74164 data sheet

# Sample C Code

This C++ code demonstrates how to control the outputs of the 4 registers via the parallel port under Win95.

LPT_Expander.hpp

#include <windows.h>

#include <conio.h>

#define LPT1 0x378

class LPT_Expander

{

private:

public:

LPT_Expander();

~LPT_Expander();

void OutputA(char Data);

void OutputB(char Data);

void OutputC(char Data);

void OutputD(char Data);

void ShiftInA(BOOL Data);

void ShiftInB(BOOL Data);

void ShiftInC(BOOL Data);

void ShiftInD(BOOL Data);

};

LPT_Expander.cpp

#include "LPT_Expander.hpp"

LPT_Expander::LPT_Expander()

{

}

LPT_Expander::~LPT_Expander()

{

}

void LPT_Expander::OutputA(char Data)

{

int i;

// Shift in the data byte one bit at a time

for (i=0; i<8; i++)

{

ShiftInA(Data & 0x01);

Data >>= 1;

}

}

void LPT_Expander::OutputB(char Data)

{

int i;

// Shift in the data byte one bit at a time

for (i=0; i<8; i++)

{

ShiftInB((Data & 0x01));

Data >>= 1;

}

}

void LPT_Expander::OutputC(char Data)

{

int i;

// Shift in the data byte one bit at a time

for (i=0; i<8; i++)

{

ShiftInC((Data & 0x01));

Data >>= 1;

}

}

void LPT_Expander::OutputD(char Data)

{

int i;

// Shift in the data byte one bit at a time

for (i=0; i<8; i++)

{

ShiftInD((Data & 0x01));

Data >>= 1;

}

}

void LPT_Expander::ShiftInA(BOOL Data)

{

char CurrentPortState;

// Capture the initial state of LPT1 so we don't corrupt it

CurrentPortState = _inp(LPT1);

// Send falling edge to register to begin cycle

// (RegisterA's CLK is Bit 0 of LPT port)

CurrentPortState &= 0xfc;

_outp(LPT1, CurrentPortState);

// Setup the data to be shifted in

// (RegisterA's Data is Bit 1 of the LPT port)

if (Data)

{

// Shift in a logic 1

CurrentPortState |= 0x02;

_outp(LPT1, CurrentPortState);

}

else

{

// Shift in a logic 0

CurrentPortState &= 0xfc;

_outp(LPT1, CurrentPortState);

}

// Give Shift register A a rising edge to clock in our data

CurrentPortState |= 0x01;

_outp(LPT1, CurrentPortState);

}

void LPT_Expander::ShiftInB(BOOL Data)

{

char CurrentPortState;

// Capture the initial state of LPT1 so we don't corrupt it

CurrentPortState = _inp(LPT1);

// Send falling edge to register to begin cycle

// (RegisterB's CLK is Bit 2 of LPT port)

CurrentPortState &= 0xfb;

_outp(LPT1, CurrentPortState);

// Setup the data to be shifted in

// (RegisterB's Data is Bit 3 of the LPT port)

if (Data)

{

// Shift in a logic 1

CurrentPortState |= 0x08;

_outp(LPT1, CurrentPortState);

}

else

{

// Shift in a logic 0

CurrentPortState &= 0xf7;

_outp(LPT1, CurrentPortState);

}

// Give Shift register B a rising edge to clock in our data

CurrentPortState |= 0x04;

_outp(LPT1, CurrentPortState);

}

void LPT_Expander::ShiftInC(BOOL Data)

{

char CurrentPortState;

// Capture the initial state of LPT1 so we don't corrupt it

CurrentPortState = _inp(LPT1);

// Send falling edge to register to begin cycle

// (RegisterC's CLK is Bit 4 of LPT port)

CurrentPortState &= 0xef;

_outp(LPT1, CurrentPortState);

// Setup the data to be shifted in

// (RegisterC's Data is Bit 5 of the LPT port)

if (Data)

{

// Shift in a logic 1

CurrentPortState |= 0x20;

_outp(LPT1, CurrentPortState);

}

else

{

// Shift in a logic 0

CurrentPortState &= 0xdf;

_outp(LPT1, CurrentPortState);

}

// Give Shift register C a rising edge to clock in our data

CurrentPortState |= 0x10;

_outp(LPT1, CurrentPortState);

}

void LPT_Expander::ShiftInD(BOOL Data)

{

char CurrentPortState;

// Capture the initial state of LPT1 so we don't corrupt it

CurrentPortState = _inp(LPT1);

// Send falling edge to register to begin cycle

// (RegisterD's CLK is Bit 7 of LPT port)

CurrentPortState &= 0xbf;

_outp(LPT1, CurrentPortState);

// Setup the data to be shifted in

// (RegisterD's Data is Bit 8 of the LPT port)

if (Data)

{

// Shift in a logic 1

CurrentPortState |= 0x80;

_outp(LPT1, CurrentPortState);

}

else

{

// Shift in a logic 0

CurrentPortState &= 0x7f;

_outp(LPT1, CurrentPortState);

}

// Give Shift register D a rising edge to clock in our data

CurrentPortState |= 0x40;

_outp(LPT1, CurrentPortState);

}

For more information on PC parallel ports and applications see this excellent site: http://www.doc.ic.ac.uk/~ih/doc/par/.

(This is were I stole the pin out of the LPT port from so I hope they don’t sue me.)